Adapting the Macintosh and Other Graphical User Interfaces for Blind Users

Alistair D. N. Edwards and Ian J. Pitt

1. Introduction

The advent of computers has been a positive development for many visually

disabled people. With suitable adaptations, many computers and applications

accessible to such people, so that, asScadden (1984) pointed out, many

visually disabled people have attained a degree of equality in job

opportunities in areas involving the use of information technology, such as

word processing. Screen enlargement, synthetic speech screen readers and

'soft' braille displays all work well with text-based computer interfaces.

However, problems have arisen with the advent of more visually complex user

interfaces, alternatively known as GUIs (graphical users interfaces) or

WIMPs (window, icon, mouse and pull-down menus). No longer is the

interaction based solely on text (typed on a keyboard and displayed on a

screen), but there are graphical items on the screen at which the user must

point, usually by way of a mouse pointing device.

The Apple Macintosh was the first microcomputer to make this style of

interaction available in the mass market, but since its release, in 1984,

the concept has become so popular that it is an option on most modern

computers and is the only means of access to many. The implications of this

for blind people were summed up by Bowe (1987), thus: 'When Drexel

University required every freshman to buy a Macintosh+ it was sending a

message that "No blind person need apply here"'. (p. 54).

This paper discusses two pieces of software which attempt to address this

difficult problem. Neither of them represents a complete solution to the

problem, but between them they give good indications of directions in which

to proceed. One thing they have in common is that they are based on the use

of sounds to replace the normal visual computer output. The first program

is Soundtrack, which was a prototype Macintosh word processor, developed by

one of the authors of this paper (Edwards). Soundtrack was intended merely

as a vehicle for testing the ideas. A much more ambitious approach was

taken by Berkeley Systems in the development and release of Outspoken, a

commercially available piece of software which attempts to make a large

range of Macintosh software accessible to blind users, through the use of

synthetic speech.

This paper presents a brief introduction to these two pieces of software

and to the principles behind them. It then gives the results of evaluating

them and draws out lessons from their successes and failures.

2. Soundtrack and Outspoken: principles

It is sometimes difficult to convey the magnitude of the problem of making

graphical user interfaces accessible to blind people especially to someone

who has not used such an interface. A large part of that difficulty is to

describe the interface and style of interaction in words. There is not the

space to attempt to describe the form of the visual interaction which is to

be adapted. It will therefore be assumed that the reader has some

familarity with the style of interface in question.

Developing a means of adapting existing software for blind users (the

approach embodied by Outspoken) is very different from developing

customized non-visual programs (such as Soundtrack). Implicit in the former

approach are certain constraints which are inherited from the visual

software and which cannot be avoided. One of the major differences between

Soundtrack and Outspoken is that because Soundtrack was more of a

customization than an adaptation, the constraints on its design were less

restrictive. There are a number of conditions and conventions which are

built into Macintosh software which Outspoken has to cope with, but which

Soundtrack simply avoids. For instance, Soundtrack uses a screen layout

which is entirely unconventional and is based strictly on a grid

arrangement, whereas Outspoken has to cope with whatever layout the

application designer has chosen - within the constraints of Apple's Human

Interface Guidelines (Apple, 1987) - and visual appropriateness.

Interestingly Outspoken does also operate as if the screen is laid out in a

grid, in that it incorporates keys which move the cursor up, down, left and

right. (Although, as mentioned in the Section on the evaluation of

Outspoken, this can cause confusion, since the screen does not necessarily

have such a layout).

The interaction between the mouse, the cursor and the other items on the

screen, controlled through the application of hand-eye (psycho-motor)

coordination, is so inherently visual that it presents a fundamental

barrier which must be overcome in order to make graphical user interfaces

accessible. Here again, Soundtrack and Outspoken represent very different

approaches. One obvious approach would have been to build a direct auditory

analogue of the visual interaction, such that as the user moves the mouse

the identity of the items to which the cursor points is spoken. However,

this is not a practical approach. The spoken feedback is slow and

confusing. This idea was therefore (effectively) rejected in both designs.

Paradoxically in this case the design of Soundtrack is more constrained in

that its design embodies the assumption that use of the mouse cannot (or

should not) be avoided and must be accommodated. Thus the user of

Soundtrack must use the mouse, but instead of speech, he or she receives

very rapid (though possibly imprecise) spatial information in the form of

tones. The pitch of the tone rises as the cursor moves from left to right

and from the bottom to the top of the screen. At any time the user can also

get spoken confirmation of the cursor's position by pressing the mouse

button.

The idea of the item under the cursor being spoken was not entirely

rejected in Outspoken, but what is different is that the user does not

control the cursor with the mouse, but through keys on the keyboard. The

cursor can be made to go up, down, left and right, and as it does so

whatever graphical items it crosses are described or read out in synthetic

speech.

Both Soundtrack and Outspoken are based on the use of sounds. Both use

synthetic speech (indeed, they use the same speech synthesizer, Macintalk),

but Soundtrack makes much greater use of non-speech sounds. Outspoken's use

of non-speech sounds is confined to simple 'beeps' marking the edges of

items (such as the last item in a menu).

3. Evaluation of Soundtrack

Soundtrack was evaluated with the assistance of eight volunteers. All were

proficient typists and all but one had experience of word processing. Their

training in the use of Soundtrack was carried out individually, based on a

set of graded exercises.

Data were obtained in several forms during the evaluation. Soundtrack

included a feature by which it was possible to record all the interactions

of the user (key presses, mouse movements and clicks etc). This facility

was used in order to investigate the nature of the hand-ear coordination

involved in interaction between a mouse and an auditory screen, by taking

measurements of the time taken to locate particular objects. Complete

records were made of the volunteers working on the final exercise, which

was the most difficult. The records took the form of a trace of the

volunteers' interaction with the program, and an auditory recording of

their verbal interaction with the trainer. Finally, after completion of the

training, each subject was given a structured interview, based on a

questionnaire.

One of the results of the structured interview was to show that all the

subjects were able to remember the layout of the auditory screen, but that

none of them could recall the formats of the individual windows. However,

it was also observed in the exercises that often subjects did not rely on

the speech output; they would move to an object using the tones to

navigate, and immediately activate it without first checking through speech

that they were in the right object. In most cases they did not make any

errors when operating in this way.

One comment made by almost all the subjects was that they would have liked

some form of physical frame within which to move the mouse. It seemed that

they found it somewhat disconcerting to interact with something which is

embodied entirely in sound. There would, however, be practical problems in

providing such a frame. Much of the power of a computer display (visual or

auditory) is in its dynamic properties; in the way the contents of a window

can be completely altered instantaneously, for example. This would not be

easy to achieve with any mechanical device.

The use of simple pitch variations to provide spatial feedback does not

seem to have been very successful. Most of the subjects did not use the

pitch of the tones to aid their navigation; they simply counted the number

of tones sounded. However, none of them found the different pitches a

disadvantage. One person did use the pitch information. He had a musical

background and located objects by listening for their particular tone.

The broad conclusion which can be drawn from the evaluation was that

Soundtrack is a usable program. Visually disabled people who were

experienced at using word processors, were able to carry out quite complex

tasks using Soundtrack with a few hours training. The one subject who did

not have previous experience did take longer to learn to use the program,

but given that opportunity he became its most skilful practitioner.

However, there were a number of problems in using Soundtrack. For the most

part these problems were caused by the high memory load imposed on users.

This was demonstrated both in the interviews and in the subjects' behaviour

during the exercises. As well as learning the mechanics of how to use the

program, they had to remember a lot of spatial information. Whereas a

sighted user of a visual graphical user interface program has ever-present

visual cues and therefore relies mainly on recognition, the user of

Soundtrack has to use extensive recall.

4. Evaluation of Outspoken

Evaluation of Outspoken has been somewhat less formal. The following

observations are based on the experiences of two blind testers. Both are

experienced computer users, one a regular user of Soundtrack and one an

experienced PC user. The comments in this section may seem overwhelmingly

negative, so the point should be made firstly that the development of

Outspoken was a highly significant step in the adaptation of computer

interfaces. Given the constraints imposed by the designers' ambitious aims

it is a laudable enterprise and an important development.

For the most part, the shortcomings of Outspoken are caused by aspects of

the visual software which it attempts to adapt, which do not lend

themselves to translation into an auditory, speech-based presentation. One

example is that speech is use both to 'read out' text displayed on the

screen and to describe graphical objects thereon. A particular icon may be

represented by Outspoken saying 'System Folder', but that is also the name

of the icon, as printed under it, which will be read out in a similar

manner. It is very difficult in such circumstances to distinguish whether

the cursor is pointing at an icon or a piece of text.

Another serious problem is caused by the assumption implicit in Outspoken

that objects on the screen are arranged in horizontal rows. Thus, if the

user presses the Cursor Down key, the cursor moves down and scans right.

Unfortunately many screen layouts place unrelated items on the same

horizontal level. Visually they are arranged so that no connection is

implied (being separated by a vertical line, for instance) but to the

Outspoken user it appears that they must be related.

The designers of Outspoken had to make the software 'break in' to the

Macintosh system software and interrogate system data structures to get the

information regarding what the system is doing and what is visible on the

screen. Since the system was never intended to be accessed in this way this

was a very difficult job, and one suspects that Outspoken does the best

possible job, but unfortunately there is some information it cannot access.

For instance it gives no intimation when menu entries are dimmed,

indicating that they are currently not selectable. Similarly it does not

tell the user what part of a text is currently selected.

Some of the difficulties of using Outspoken are due to inconsistencies in

the visual interface, which become more apparent when the interface is

accessed through speech. The designer of the visual interface can be

slightly more relaxed about adhering to rules of consistency because they

can ensure that there are ever-present prompts and cues visibly available

to the user. In general, this is not true of an auditory interface, at

least not of Outspoken. One exception is that when the user is 'dragging'

an object, there is a constant tone to remind them of the fact that they

are in a dragging mode. There is scope for further investigation to see

whether more such persistent reminders might be made available in an

auditory interface.

One screen item which is relatively easy to use through Outspoken is the

menu. Hence, it is a good idea to treat as many objects as possible as

menus. It is possible to adjust some windows so that they operate in a

manner very similar to menus. However, this can also cause problems because

although windows can be treated in a similar manner to menus, there are in

fact differences and the user can become confused. Again, the difference

for the sighted user is that they can clearly see when they are interacting

with a window or a menu and there is a clear visual difference between

them.

5. Lessons: further work

One immediate lesson to be learned from the experiences with Soundtrack and

Outspoken is that there is still a significant problem to be addressed.

Neither of these approaches solves it completely, both are just steps along

the way. Some of the lessons will apply to the design of auditory

interfaces, while some of them are broader, relating to human-computer

interaction generally, and to how interfaces can be made more adaptable.

One obvious need is for access to high level descriptions of events in the

human-computer interface, so that these may be manifested in more than one

way. For instance, some Macintosh events are confined to the manipulation

of pixels on the screen. This cannot be accessed or interpreted in any

other way, and hence Outspoken is unable to communicate the event to the

user. The advent of user interface management systems (UIMs), which attempt

to separate the interface from the application, seem to offer hope in this

direction. (See Betts et al, 1987).

Both Soundtrack and Outspoken are limited in the sounds which they use.

Outspoken relies almost entirely on speech and Soundtrack's non-speech

sounds are simple and convey very little information. There is evidence

(Bly, 1982) that a lot of information can be communicated in non-speech

sounds and this fact ought to be exploited fully. Work is already in hand

at York looking at using more dimensions of sound, including stereo for

auditory navigation.

Experience with Outspoken suggests that a lot of errors and confusion are

caused by the fact that the interaction is inevitably 'modal' but the fact

that the software is in a mode is not obvious to the user. (See Tesler,

1981, and Johnson, 1990, for discussions of modality and its significance).

Investigations into the possible use of constant sounds signalling the

current state and mode are also being carried out at York.

Although both Soundtrack and Outspoken use sounds, it is possible to

conceive of adaptations based on tactile output, and experiments have been

carried out with this approach. A number of different researchers have

latched onto the idea of a 'braille mouse', a mouse which displays a

tactile braille image of the text under the screen cursor. (See, for

instance, Kalina, 1989). Another tactile approach is based upon the Optacon

reading device, used by many blind people to read printed text#. A version

is now available which can be linked to a Macintosh, such that the contents

of the screen around the cursor are reflected in the Optacon's tactile

output. Experiments have also been carried out at York into the use of

simple tactile feedback to supplement auditory and/or visual feedback

(Luff, 1990). In the future it seems likely that such combinations of

output channels will be integrated in truly multi-media interfaces, which

will be easier to use for all people, but also easier to adapt for those

with disabilities (Edwards, 1990)

6. Conclusions

One of the most significant contributions of development of interfaces such

as Outspoken in the long term, will be the way they highlight

inconsistencies and design flaws. If the designers of visual Macintosh

software were to go back and re-design their software to be more compatible

with Outspoken - in terms of consistency alone, regardless of visual

presentation - they would probably produce software which is also easier

for sighted people to use.

Outspoken is a very worthy attempt to address a very difficult problem.

Unfortunately that problem was just too complex for it to succeed

completely; the availability of Outspoken will not persuade large numbers

of blind people to buy Macintoshes. Nevertheless, the development of

Outspoken will prove in the future to be very significant as it has

highlighted exactly what is difficult about adapting software with a

graphical user interface. Software designers in the future will hopefully

benefit from those lessons and make programs which are both usable by the

able-bodied majority and easily adaptable to make them accessible by users

with disabilities.

7. References

Apple Computer Inc. (1987), Apple Human Interface Guidelines: The Apple

Desktop Interface, Addison-Wesley, Reading Massachusetts.

Betts, B., Burlingame, D., Fischer, G., Foley, J., Green, M., Kasik, D.,

Kerr, S. T., Olsen, D., & Thomas, J. (1987), Goals and objectives for user

interface software, Computer Graphics, 21, pp. 73-78.

Bly, S., (1982), Sound and computer information presentation, PhD Thesis,

University of California, Davis, California, USA.

Bowe, F., (1987), Making computers accessible to disabled people,

Technology Review, 90, (1) pp.52-59.

Edwards, A. D. N., (1987), Adapting user interfaces for visually disabled

users, unpublished PhD Thesis, the Open University, UK.

Edwards, A. D. N.,  (1990), Redundancy and adaptability, in Edwards, A. D.

N and Holland, S. (eds.) Multi-media Interfaces and Learning, Springer-

Verlag, Heidelberg (in press).

Johnson, J., (1990), Modes in non-computer devices, International Journal

of Man-Machine Studies, 32, pp. 423+438.

Kalina, U., (1989), Braille window mouse: proposal for a new computer

output device for the blind, Paper presented at the Workshop on Access to

Computer Systems by Blind Persons (including Windows, Icons, Mice and Pull-

down Menus), Madrid, (Proceedings in preparation).

Luff, R., (1990) Development of an interactive mouse,  unpublished

undergraduate project report, Department of Computer Science, University of

York.

Scadden, L. A., Blindness in the information age: equality or irony?

Journal of Visual Impairment and Blindness,  (November 1984).

Tesler, T., (1981), The Smalltalk Environment, Byte Magazine, August, pp.

90-147.

8. Acknowledgements

Grateful thanks are due to the volunteers who carried out the evaluations.

Specifically we would like to mention Alistair Williamson and Chris

Bartlett. Thanks also to Corri Barrett of the RNIB and Karl Farrell of

AVHOW for organizing volunteers.

Apple and Macintosh are registered trademarks of Apple Computer, Inc.

outSPOKEN is a trademark of Berkeley Systems, Inc.