New Perspectives in the Economics of Braille Production

Prof.Dr.Ir.G.Franois

Braille production has the reputation of being expensive. In this paper I

want to examine how much the recent technological advances can alleviate

that problem. I will do so in particular by referring to the results of our

own work on computerised Braille editing and electronic Braille embossing.

The production of hard-copy Braille comprises a number of phases :

1. The input of source text

2. The translation into Braille, perhaps higher-grade Braille

3. The formatting

4. The verification and correction

5. The embossing

6. The sorting and binding.

In the case of manual embossing, the operator simultaneously performs the

inputting, translation, formatting and embossing. Verification and

correction must be done subsequently. There is no sorting as there is only

one copy. Binding can be done in a number of ways.

Manual embossing requires a minimal investment of about 800 ECU for a

Perkins machine. A book of 500.000 characters - the standard size which we

will consider in this paper - will require about 140 operator hours

(roughly one month's work), equivalent to 1.250 ECU. Verification and

correction require about half that much. A good binding by manual stitching

will take at least another 100 ECU. The total cost of this book will thus

come close to the impressive figure of 2.000 ECU, about 200 ECU per Braille

bundle. The cost of the Braille paper is only a neglible part of that sum.

The production of a zinc plate master is done nearly along the same line as

the manual embossing. A motorised press replaces the Perkins brailler. The

operator time will be roughly twice as high. Verification and correction of

the plates is particularly difficult because of the two-sided embossing. By

the time we are ready for printing, the zinc master for our standard book

will have cost almost 4.000 ECU. A good press will then print copies at the

net rate of 1.000 or more sheets per hour. We have to sort these out to get

Braille bundles. The incremental or so-called marginal cost per book will

strongly depend on the efficiency of the operator, but in any case, the

cost of printing and sorting will get down to within a factor of 2 of the

cost of the Braille paper, let us say 10 ECU per copy of the book. Unless a

very simple binding is used, such as stapling in the middle of the sheets,

the total marginal cost is likely to be dominated by the cost of the

binding. An investment of the order of 50.000 ECU is needed for the presses

used in this method, but the writing-off period for such mechanical

apparatus is very long, 20 years at least.

What can computers and computer software contribute to the process of

Braille production ?  The basic new element which computers introduce is

that the data are stored on electronic carriers. The software will ease the

processing of the text. Usually that means editing, correcting and

formatting but in the case of Braille we may add applying the rules of

Braille writing and coding into higher grade Braille. The task of the

operator can thus be simplified considerably.

As a first step, we consider the manual input of text on a keyboard. Using

present-day software for text processing to Braille, the typist should need

no more than 70 hours to enter the standard volume book in the computer. At

this point, verification can be done either on the video screen or on a

printout. Correcting on the electronic carrier should be a straightforward

procedure.

The investments for these steps may be as low as 2.000 ECU per work station

and the cost for inputting a standard-size book will drop to about 1.000

ECU.

Even for single copies, the gain is substantial and the investment pays off

over only a few books provided we can find an appropriate output device.

The gain becomes even more compelling as soon as we make multiple copies of

each book.

We can go a lot further on the input side and avoid the costly typing task

by using either editor's files or optical character recognition of

inkprint. The first technique is the most straightforward one. A decade

ago, it was rather difficult to apply because of the many different

standards used by editors' apparatus. In recent years however, the wide-

spread use of "compatible" PC's using the MS-DOS operating system, has

opened the way for an easy exchange of information on standard floppy disc.

In the near future, further development of electronic networks should make

the flow of information still easier. It is an urgent matter that a legal

framework be set up on a world-wide scale to make this flow possible and

define exactly the rights and duties of all parties involved.

Theoretically, the cost of this electronic data transfer could be very low

: in less than one hour, a simple PC will handle the entire book. However,

even the most sophisticated software will have flaws that introduce

spelling errors in the Braille text. And Braille, as you know, is very

intolerant for errors, because it is read analytically, character per

character. Therefore, the text must be proofread carefully and corrected.

It will take hours and even days to do this job. The choice then is :

either Braille the "quick-and-dirty" way or high quality editing at a

certain price. For good work, this price is likely to fall in the order of

250 ECU for our standard book. For work of lesser importance, the cost may

be close to nothing.

For those cases, where no access to an electronic carrier is available, the

OCR technique - optical reading - may be applied. Important progress has

been made in recent years through the combination of OCR software with

spelling checking. Some users report close to 99,9% error-free reading.

That means about one error per Braille page : an acceptable limit. The

investment involved is important : of the order of 25.000 ECU. The cost per

standard-size book will be anywhere from 100 to 300 ECU, depending on the

quality of the inkprint edition, the performance of the software used and

the care given to the work by the operator.

In summary, we can see that the use of computers will reduce the

acquisition cost of the text by a factor 2 at least, more likely by a

factor 8, even for high quality editing and possibly much further, down to

very little if we have access to the editor's files and use a good software

program. At this point I should mention PCBraille which has been developed

in our group and which has a widespread use in several EEG countries. Mr.

Mennens, who is the principal author of this program will give more details

about it in his paper.

Having the text properly prepared, we must now consider the embossing.

For large numbers of copies we can resort to the automatic, computer-driven

production of master plates in zinc, aluminum or plastic. This requires an

investment of about 100.000 ECU. From that point on, we can print copies as

we used to, by mechanical press.

For single copies or medium numbers of copies, electronic embossers can be

used. Their prices range from 4.000 ECU for a personal character embosser

up to as much as 80.000 ECU for certain two-sided embossers. Our own

single-sided Elekul line embosser fits in there at the 20.000 ECU level.

These machines have advantages similar to the ones photocopiers have over

stencyl machines : they can make multiple copies just as efficiently as

single copies. No sorting is needed as complete books are sent out

sequentially by the computer.

Character printers are slow, giving an output of 10 to 40 characters per

second. They are generally not designed for large volumes of work. Line

embossers give outputs ranging from 200 to 400 characters per second,

giving up to 1.000 pages per hour as does the Elekul.

The cost of running an electronic embosser will be determined by the type

of paper used, the production efficiency, the operator's time for the

handling after embossing, the maintenance cost and the depreciation or the

life time of the unit. We believe that the Elekul embosser scores very high

on each of these points and for this reason, we will refer to it for the

further calculation of cost figures.

In the first place, the Elekul is the only unit on the market, that uses

paper from rolls, without sprocket holes and without folds or perforations.

Pin fed paper would be more expensive by a factor of 2 or more, thus

offsetting any possible cost advantage of two-sided printers.

Second, the operator's time is minimal because the unit cuts the pages to

size after embossing : the bundle delivered by the machine is ready for

binding. The built-in cutter also allows a free choice of the page length,

which can be another element of savings.

Third, the maintenance cost has proven to be exceptionally low. This is

mainly due to the mechanical sturdiness and the low wear of the printing

block.

Excellent results with first class durable relief can be obtained with

paper at a price of 15 ECU per 1.000 sheets. That would put the cost of our

standard book at 10 ECU.

Adding the operator's time brings this to 17 ECU. Experience shows that the

maintenance cost will never exceed 1/3 of the cost of the paper and

therefore we get our book for the marginal cost of 20 ECU. This is twice

the price we figured for the technique of zinc plates but remember, how

high the cost of these masters could be. And think of the work needed for

sorting and the cost for the storage of the edition and its masters plates.

Also think of the speed of production : if we receive a book on diskette,

we can have it in Braille one hour later by the electronic way. It would

take days to make the masters, run them on the press and sort out the

resulting piles of paper.

There are several examples of centers using the Elekul outfit for the

routine production of periodicals on a monthly, weekly or even daily basis.

I will give you some facts about the latter example.

The newspaper "Est Rpublicain" which is edited and printed in Nancy,

France has recently started a Braille edition. The articles printed are the

same as those appearing in inkprint. Each night, the redaction composes the

newspaper. The articles chosen for the Braille edition arrive on floppy

discs. Within 20 minutes, they are reformatted for Braille and the actual

embossing takes a start even before the rotary inkprint presses get

running. Each copy has some 30 pages of 1.100 characters and there are

about 250 subscribers. Two Elekul embossers run the edition between 11 PM

and 4 AM and the blind reader gets his newspaper at the same time as his

sighted neighbour and at the same price. This price covers the cost of the

paper. The cost of the operator and the maintenance of the hardware do not

exceed the cost of the paper and are covered by the advertising budget.

As you can see, under optimal circumstances, electronics can drastically

reduce the cost of Braille printing and - perhaps more important -

eliminate the time lag which has been so characteristic of Braille

production in the past.

Table 1 summarises a few of the results we have mentioned. It compares the

startup cost and the incremental cost for a standard book as well as the

investment needed for various techniques, classical ones and electronic

ones.

The effect of startup cost and incremental cost on the total cost of an

edition can be seen in Figure 1.

From the table and from the figure, we can draw the following interesting

conclusions :

1. In classical production, the break-even point between manual embossing

and zinc presses is reached very soon when multiple copies need to be made

: the zinc master pays off as soon as a third copy is made. The press will

be paid for after as few as 25 third copies. This fact is well recognized

in practice.

2. For single copies, the keyboard input and PC driven embosser should

always be preferred over the manual embossing. There is no break-even point

in this comparison. The investment needed pays off after as few as 20

books. The continued routine use of a manual embosser in an industrialised

continent as Western Europe, is a waste of time and money. This fact is

slow to be recognized as yet.

3.For somenone who has at his disposal both an electronic embosser and a

classical zinc press, the break-even level for going to zinc is situated at

300 copies, give or take say 50 copies. This is a rather unexpected high

figure.

4. In the case of electronic master production this break-even level comes

down to 35 copies but the investment for the use of masters is important

and will only pay off after 8.000 copies beyond the 35th are made (80 .000

Braille bundles). In addition, there is virtually no break-even level for

the production time : the electronic embosser gets a head start of about

one day and the press will never catch up with it.

As a final point, let us have a look at the future. We are working now on

the development of a two-sided embosser which should have the same

favorable characteristics as our present machine and be even more economic

up to the point where it can advantageously replace the Braille presses,

even for large editions. It is designed for transverse printing - a

technique which we have proposed long ago, in 1982 already - and should

allow the production of double pages that can be bound into quires by

simple stapling.

This effort and similar ones notably in the domain of the software for

Braille coding and formatting, by our group and by others as well, should

bring us step by step closer to the ultimate goal where information will be

available to those who need it, in the way that suits them best and

virtually at the same price and the same time as for any other person.