Software applications in the communication between a computer and its blind

user as well as in the modernization of braille production based on the

latest developments in Poland

Helena Jakubowska, Stanislaw Jakubowski

A computer cannot be considered as a suitable and sufficient aid for a

blind person, unless the user is provided with an efficient system of

communication with this sophisticated tool. To achieve this goal it is

necessary to comply with the following terms:

1. The blind user should be capable of entering all possible data.

2. The widest possible range of data displayed on the screen should be

accessible to the user.

One of the ways to solve the problem mentioned in item 1. consists in

making a program by which selected keys of the standard computer keyboard

take the function of the brailler keyboard. A short description of such a

program has been given.

The second condition i.e. making the information from the screen accessible

to the blind user can be fulfilled by the following means :

- Synthetic speech.

- Magnified characters on the screen.

- Optacon.

- Braille display.

In Poland the first two techniques are more applicable due to their lower

cost. The most important features of programs serving the above purposes

have been discussed in this paper.

Another significant field of application of computer techniques is braille

production. The most important functions of a software package worked out

for this domain have been described. The paper ends up with a short

presentation of methods of publishing large print periodicals. This report

summarizes all significant results achieved in the field of computerized

braille and large print production by various Polish authors since

Winterthur conference in 1986.

Since the previous conference in Winterthur during which I was presenting

some investigations on computerized braille production started in Poland I

have not been able to demonstrate any applications of this work so far.

Therefore, it is a great pleasure for me to take this opportunity to report

that the Printing House of the Polish Association of the Blind has been

successful in employinga computer techniques in the production of braille

and large print periodicals and books In addition, pioneer home-production

devices and programs designed to facilitate the blind or partially sighted

user's communication with a computer, have been introduced.

Considering the growing applicability of computers in the work of and for

the blind one can notice that in many countries computerization was

originally introduced in braille printing houses and only after several

years gradually reached individuals. It was long before first blind users

could utilize their own computers at home that the total blind community

were provided with books and magazines prepared with the use of computer

techniques and modern braille printers. In Poland the situation is similar,

although we cannot clearly distinguish between the two following stages,

since before it was possible to computerize our printing house, a team of

programmers managed to prepare first packages making a computer available

to the user with impaired vision.

For many blind users it becomes rather difficult to get well- acquainted

with the complicated computer keyboard. Some, however, are quite efficient

in using a brailler and it is for them that a resident program called

Keybrl was written by one of our blind programmers, I. Buslowicz, making it

possible to enter data in the six- or eight-dot braille notation. There are

several characteristics distinguishing this program among a group of others

of the same type. A blind person may alternately use the standard computer

keyboard or a set of selected keys taking the functions of brailler keys

and thus corresponding to particular dots. To switch over to either of the

two keyboard functions you simply press an alt-space keystroke. In the six-

dot notation the number and capital signs can be used without any

limitations. The pressing of either of these dot combinations does not make

any additional symbols appear on the screen and their conversion into the

standard system takes place automatically. Thus, for instance, if the

letter "a" is preceded by the number sign, the number 1 will be displayed

on the screen. If, however, we put the capital sign in front of the letter

"a", we will see it on the screen as "A". The use of "r" parameter while

calling Keybrl program provides the user with a possibility to write with

the right hand only. The parameter "l" has been designed to switch over to

the left hand braille keyboard. A modified version of keybrl program has

successfully been applied as a keyboard driver to the German production

Blindenstudienanstalt printing machine and due to this improvement blind

copyists who used to work with printing machines of this type can retain

their previous manner of writing while using electronic braille devices.

Another necessary condition for the user with impaired vision to be able to

work with a computer is that he or she should be able to perceive all the

information displayed on the screen. In Poland, where computers have been

accessible to blind individuals since not long ago, software techniques are

obviously preferable in solving this problem. Thus, for example, Lens

program designed to magnify characters on the screen does not require any

hardware additions or modifications. Lens, prepared also by I. Buslowicz,

is capable of magnifying a fragment of text consisting of 5 or 6 lines, 10

characters each. The user can choose among three magnification ranges: 3x,

5x and 10x. One of the serious limitations of the program, however,

consists in the fact that it is suitable for the text mode only.

Similarly to the talking book being the most popular traditional form of

access to literature, a speech synthesizer is becoming the most preferable

computer aid among the blind in Poland. Two models of the device are

available: one applicable to IBM PC and the other adapted to ZX Spectrum

class computers. The synthesizer is connected to the Centronics output and

employs the central processor of a computer. A so called sampling synthesis

has been used here. Polish phonemes have been stored on a floppy disk as a

file. With the use of the synthesizer we can not only obtain all the

information from the screen but also get an acoustic representation of

every key stroke. This synthesizer is suitable for the most popular text

editors such as WordPerfect, WordStar or Word. The synthesizer, including

software, costs less than 200 dollars. Phonemes have been electronically

recorded in a digital form by Jan Grebecki, a skilful technician, while the

synthesis and software necessary for the user have been made by one of our

blind programmers, Marek Kalbarczyk.

The modernization of our printing house was undertaken in 1988. Braille

printers: Puma-V and Braillo 400 have been imported to publish braille

periodicals and books. All the necessary software has been prepared by

blind programmers, I. Buslowicz and S. Jakubowski within projects carried

out by the Laboratory of Computer Science at the Polish Association of the

Blind. Nearly all the traditional German printing machines have been

removed from our printing house and replaced by computers. Since most of

our copyists are visually impaired, it has become necessary to provide them

with braille displays. Blind transcribers with their thorough knowledge of

the braille system are entrusted with all difficult texts, particularly

those requiring the use of special braille notations such as the

mathematical, musical or phonetic one. Sighted transcribers are most

frequently given texts assigned for prompt publication, e.g. periodicals.

Both groups of copyists, however, enter texts under Wordstar Professional

since this text editor complies with the following terms:

a. The editor works in the character mode allowing for the application of

braille displays or speech synthesizers, without which the employment of

blind transcribers or proof-readers would not be possible.

b. There is a special program called Wschange which makes it possible to

arrange the edited text according to a braille page size.

c. Wordstar Professional allows for the entering of your own instructions

under so called dot commands or four user's commands. This trait is

particularly helpful when we want to introduce some conventional

instructions into a braille page formatting program.

Brajl program designed in 1989 is constantly being improved. Its main

functions include: the conversion of texts written under Wordstar

Professional as well as ASCII files into braille, the control of Puma-V and

Braillo 400 printers for such parameters as page size, spacing between

braille lines, page numbering, number of copies etc. The program is also

responsible for word hyphenation, making tables of contents, proper layout

of footnotes and tactual representation of inkprint signs by braille dots.

This year our printing house has started publishing large print

periodicals. Both, large print and braille versions are obtained from the

same data stored on a diskette. text files are processed by the Polish

Desktop Publishing type program called Cyfroset, which provides for the

arrangement of the large print text. Photographs have to be glued onto

paper. Drawings in black and white are read by the scanner and incorporated

in the text by the above mentioned Cyfroset program. The original of the

text is prepared with the use of a laser printer on a special kind of

transparent sheets. It is used then as the source material for offset

printing.

To conclude, it is my strong belief that this short review of the latest

developments in the field of computerization to the benefit of the Polish

blind community has highlighted considerable progress marked during the

last few years in our country. Some of our experiences may appear useful

for those countries who have just started the process of applying computer

science and microprocessor-based devices in the rehabilitation and

education of the blind.