Permanent and temporary tactile graphics for blind computer users

Gunnar Jansson

For the reading of pictures, blindness means that variations in light

intensity and colour over the surface of a picture are not useful.

Information in some other form must be available. Description in words is

an often used substitution, but this variant has severe limitations; a

picture tells you more than a thousand words.

There have been efforts to use some auditory code as a replacement, but

this is applicable only to very simple pictures and there has hardly been

any success even in these cases. The most successful undertakings of making

pictures available to the blind have replaced the variations in light

intensity with variations of embossment over a mainly two-dimensional

surface.

In some instances, such as the Tactile Vision Substitution System (see,

e.g., Bach-y-Rita, 1972), the picture is produced on a matrix constantly

covering some skin area, but in most cases the picture has to be picked up

piecemeal by the finger tips.

1. Some general problems

Even if the finger tips are among the most sensitive skin areas of the

human body and well suited for the task of picking up information by moving

over a surface, there are some problems that seem to be unavoidable (a more

detailed discussion can be found in Jansson, 1988).

A well-known problem for anyone who has tried to read a picture with the

finger tips alone is the getting of an overview. In contrast to vision,

which presents a nearly immediate overview of pictures of ordinary size, a

picture examined tactually has to be searched during an often rather long

time before the observer has got an idea of its general character. It also

happens that the reader of a tactile picture gives up without having got

any information at all from the picture.

Not only overview, but also details are common problems in tactual

pictures. In spite of the finger tips in many contexts being very acute in

picking up information from surfaces, patterns produced in our present

forms of tactual pictures are often difficult to discriminate from each

other. This makes it hard to use as many different kinds of surfaces as you

would wish and necessitates elimination of details and other

simplifications of a tactile picture as compared with the corresponding

visual picture.

A third difficulty with tactile pictures is the reading of three-

dimensional properties included in the picture. There is a theoretically

very interesting dualism in a visual picture in its being a two-dimensional

surface and its containing three-dimensional objects - at the same time.

This perceptual dualism is difficult, some say impossible, to obtain in a

tactile picture. Typically, what you perceive when examining a tactile

picture is a pattern on a two-dimensional surface.

Even if some problems always will be with us, for instance the difficulty

of getting an overview, other problems vary with the kind of tactile

picture used. One important property is the spatial resolution of the

picture. If it is high more details can be perceived. Low spatial

resolution means that many details can not be reproduced and that forms may

be distorted. The usefulness of tactile pictures varies widely between

different versions, which means that any judgment about their suitability

has to be considered for each variant separately.

2. Graphics for blind computer users

In the case of personal computers, the technical development has meant

advantages also for the blind. These computer users can pick up

alphanumeric signs with the aid of synthetic speech, patterns on an Optacon

or as braille patterns on paper or on tactile point displays. Computer

graphics, visually simple symbols as well as more complicated pictures, is

a much harder challenge, at present to a large extent unavailable to the

blind computer user. This has several drawbacks, such as not being able to

utilize software including graphics and being excluded from all the

computer pictures which increasingly are made available visually via, for

instance, CD-ROM drives and video cameras. This means that the blind

computer user, among other things, has to restrict himself to the

alphanumeric content when using an encyclopedia on CD-ROM.

The aim of a research project in progress (cf Jansson, 1990) is to

investigate the possibilities of making pictures available for the sighted                                                                        2

available also for the blind, at least to a higher degree than now. In this

work, a main task is to make comparisons between different kinds of tactile

displays, permanent ones corresponding to printers and plotters for visual

pictures, as well as temporary ones equivalent to a computer screen.

3. Permanent and temporary tactile options

In most cases, permanent tactile pictures are produced by generating

embossments on paper or plastic sheets. A versatile method of doing this is

to print or plot the picture in the usual way and then copy this picture,

on an ordinary copying machine onto a special paper, called capsule paper

(Goto, 1983) or swell paper (Magnusson, 1986). When this special paper is

suitably heated the dark pattern on it will be embossed.

Another way of getting a tactually readable picture is to produce embossed

points with the aid of a braille printer in graphics mode. However, so far

these printers have much lower spatial resolution than the capsule/swell

paper.

All methods developed until now of producing a temporary tactile picture

are based on a matrix of point stimuli. Within this matrix a pattern

corresponding to the picture is formed by some of the stimuli being raised,

vibrating, or producing a weak electric current.

The Optacon (for its development, see, e.g., Linvill, 1973) is the most

well-known of these devices. Version II consists of 20 rows of 5, thus in

all 100, vibrators. The tactile display can be used either directly

controlled by a computer or, via its camera, for reading a visual picture

produced on paper. The largest of these matrices constructed so far

consists of 7 200 (120 x 60) pins individually controlled by a computer

(Schweikhardt, undated).

4. The picking up of texture gradients

The first series of studies in the ongoing research project mentioned

before concerns the picking up of texture gradients. One of the main

problems of tactile pictures is the perception of the three-dimensionality

of the scene (cf. above). In visual pictures, one of the main features to

produce a perceived variation in depth, for instance slant or bending of a

surface, is to change the texture successively. This successive change is

called a texture gradient. Can this feature of a picture be picked up by

the finger tips?

Some experiments I have made together with Barry Hughes demonstrate that

such features can be picked up. We produced texture gradients corresponding

to different slants and asked the subjects to judge the amount of slant

after having explored the picture with one finger. Their results were

astonishingly accurate. The same was true, to a somewhat lesser degree,

when the subjects examined the picture with an Optacon. This was an

unexpected result as there are so many dissimilarities between the two

kinds of display, for instance spatial resolution, raised points and lines

versus vibrators, and the use of the same hand for exploring and perceiving

versus the use of different hands for the two activities.

The project will continue by studying other kinds of displays, as well as

other aspects of the content of a picture.

5. Conclusion

It seems to be a reasonable expectation that tactile graphics can be made

available and found useful for blind computer users to a much higher degree

than is common at present. However, a prerequisite for this is that we can

deepen our knowledge about how the pictures should be constructed in order

to be most useful for the observer. The invention of novel kinds of

displays better adapted to the way the fingers pick up information may also

give new opportunities for the blind to appreciate computer graphics.

6. References

Bach-y-Rita, P. (1972). Brain mechanisms in sensory substitution. New York:

Academic Press.

Goto, R. (1983). New techniques for tactual graphics. In J.W. Wiedel (Ed.),

Proceedings of the First International Symposium on Maps and Graphics for

the Visually Handicapped (pp. 113-125). Washington, DC: Association of

American Geographers.                                                   3

Jansson, G. (1988). Which are the problems with tactual pictures, and what

can we do to solve them? Paper read at a workshop on "Man-Machine

Interfaces, Graphics, and Practical Applications", Maastricht, the

Netherlands, November 14-16, 1988.

Jansson, G. (1990). Tactile graphics for blind computer users. In K. Noro &

O. Brown Jr. (Eds.), Human factors in organizational design and management

- III (pp. 241-244). Amsterdam: Elsevier (North Holland).

Linvill, J.G. (1973). Research and development of tactile facsimile reading

aid for the blind (the Optacon). Final report. Washington, DC: U.S.

Department of Health, Education, and Welfare, Office of Education, Bureau

of Education for the Handicapped.

Magnusson, B. Svllpappersframstllning av taktila kartor (Swell paper

production of tactile maps). In J.-I. Lindstrm & H. Benlala (Eds.),

Taktila kartor och bilder (pp. 6-13). Enskede, Sweden: SRF Konsult.

Schweikhardt, W. (undated). Research on computer based aids for the blind.

Stuttgart: Universitt Stuttgart, Institut fur Informatik.