Braille, Moon and large print workstations developement

John Kaplin

1. Background

RNIB has been producing braille by computer assisted methods since 1969. In

October 1978, to coincide with a move to a new site, a considerably

enlarged

and more sophisticated system was installed. Ten years later similar

circumstances prompted another computer-based production system. This third

system, and the experiences that led RNIB to it form the basis of this

paper. The author was closely involved with the implementation of the mark

II system in the mid-'70s and was responsible for co-ordinating the

development of the current system.

The first computer system used by RNIB to assist in braille production

consisted of an 8k byte IBM mini-computer fed by punched cards. A volume of

braille of approximately 130 pages took about 2 hours to translate and

format. As output another pack of punched cards was produced which were

used to drive a modified manual metal plate embossing machine. An extremely

labour intensive procedure, this method of production was only used for

standard texts, most braille production was still carried out by

traditional manual methods.

In early 1977 a working party started to put together a specification for a

new system, to be installed at a new production site. The system that was

installed consisted of a twin mini-computer configuration supporting 16

VDUs on each processor.

Also included, along with the usual peripherals such as disk and tape

drives and printers, were 4 braille editing work-stations with A3 size

portrait orientation screens able to display a full page in a large braille

screen font. These QVECs were adaptations of CAD systems using a custom

built keyboard, including dedicated brailling keys, and bespoke software.

The devices were originally intended as editing stations, to be used for

final tidying-up of braille work but they proved to be the most reliable

and the most popular and were extensively used for originating braille.

Original braille input was carried out using bespoke stand-alone devices

known as BCTs (Braille to Cassette Terminals). These BCTs consisted of a

Perkins-like keyboard and a small cluster of control keys together with a

six line 40 character braille display. Output from these devices was via a

standard data cassette tape and files could be loaded onto the mini-

computer for editing on the QVECs. Unfortunately, the BCTs did not have an

editing capability, nor were they sufficiently reliable. As a result they

were the first elements of the system to fall into disuse.

Text input for computer translation into braille was via the VDUs attached

to the mini-computer. An in-house data-entry programme provided for input

and

editing. A verification facility was also available but not widely used. As

text was keyed from a source document formatting codes were inserted as

appropriate. The data was then translated into braille via a version of the

DOTSYS program running on the mini-computer. Corrections were applied

either to the braille file via the QVECs, or to the original text prior to

retranslation. Corrections to the ASCII version of the braille file were

also possible using a text editor on the VDU.

Final braille ASCII files were output from the mini-computer on data

cassettes to drive modified versions of the original plate embossers used

in the earlier card-based system. On-line production-scale braille

embossers such as the RESUS 14 and Braillo 270 were also incorporated

during the life of the system. The RESUS embossers replaced and

supplemented ageing LED 120s which had been used for short runs and

proofing copies since the early days of the system.

A Kurzweil Data Entry Machine was a later addition to the text input

process and supplemented the VDUs. With the KDEM able to handle standard

text the typists at the VDUs could deal with work demanding more

complicated formatting instructions.

As a development of these systems in 1987 micro-computers started to be

used for braille transcription and subsequently for text input. IBM

compatibles were not involved at this juncture, but BBC micros which were

used extensively in education at that time and for which suitable software,

including a braille screen font and six-key chording facility were

available. These systems provided RNIB with a good stop-gap solution whilst

a specification was developed for a replacement system.

Moon is an embossed system of characters, larger than braille cells and

more closely resembling print letters, which also has contractions for

commonly

used letter sequences. At this stage Moon was still being produced by

traditional methods i.e. Moon characters were set by hand in a similar way

to "hot-metal" in the print industry. The pages were then pressed onto wet

manilla paper to impress the shapes and then the paper was carefully dried,

an extremely laborious process.

The opportunity to modernise Moon production in line with braille and in

some way compatible with it was added to the requirements specification. It

was planned that the Moon production system would be relocated to the

Peterborough site at the same time as braille production moved.

Large Print had not featured in RNIB production but was recognised as an

area for which there was potentially a large demand. Large Print readers

are able to obtain copies of popular books through normal sources but there

was no facility for providing such things as information sheets or

brochures in appropriate font sizes. This capability became a requirement

also. At least this was something that standard software should be able to

provide.

2. Requirements Overview

The objective was to provide a work-station able to facilitate all of the

steps involved in the preparation of texts for production in any or all of

the media proposed.

The steps involved are:

- Text Input/Editing

- Braille Input/Editing

- Moon Input/Editing

- Translation of text into Braille or Moon

- Large Print preparation

- Archiving

- Networking

A major consideration was that, wherever possible, standard commercial

software and hardware should be used with the minimum of bespoke

development.

The potential benefits from compatibility with other producers of reading

materials for the visually handicapped was borne in mind in designing the

new systems. Transfer of data to other organizations for local

printing/embossing is likely to be as useful as receiving data from outside

producers.

Tactile displays and/or speech output are necessary if visually handicapped

users are to have access to the systems.

2.1. Text Input and Editing

Keyed input using a commercial package. This route would be particularly

relevant to text which is not suitable for Optical Character Recognition

(OCR).

OCR origination using the KDEM or a similar system. Editing to be carried

out using PC based text processing software. This would be primarily

concerned with preparation of text for braille and Moon translation.

Data coming from external sources e.g. compositors tapes, CD-ROM, home

volunteers, other organizations and other sources (including PRESTEL,

TELECOM GOLD and other data communications systems).

In order to facilitate maximum flexibility, wherever possible corrections

should be applied to the text file and re-translation into the appropriate

medium carried out.

2.2. Braille Input and Editing

The system should offer the options for input via a chording sequence or

QWERTY keying:

- using a software modification such as the "SIX" program to enable the

home keys of a standard keyboard to be reconfigured as the braille keys, in

the style of a Perkins Brailler, for those braillists who prefer this mode.

- using the braille ASCII character set for direct input of grade II

braille via a standard keyboard.

The display should be capable of depicting the braille either in a six-dot

matrix form or as the ASCII representation of the braille characters. The

ability to display the complete braille page would be highly desirable, as

page layout etc. are often important. An A4, or portrait orientation screen

may be appropriate.

2.3. Moon Input and Editing

In a similar manner to the braille system Moon can be input on a QWERTY

keyboard using the ASCII equivalents of the Moon characters and either a

Moon font or the ASCII generated on the display.

2.4. Translation into braille or Moon

The braille translation software should be capable of producing a reliable

close approximation to grade I and grade II Standard English Braille from

texts originating from any of the in-house sources. Integration with the

text processing software should be developed to minimise the amount of

formatting information required to be added to the text. A facility for

translating braille into print text would also be a useful facility, though

not essential.

An equivalent program for converting the same text into the appropriate

Moon codes would also be required. An earlier project under the auspices of

Dr John Gill developed a Moon translator which adhered to the rules imposed

by the use of the Boustrophedon effect (bi-directional reading facility). A

decision to no longer implement the Boustrophedon should simplify the task

of

translation.

2.5. Large Print

Text which has been prepared for tactual production routes should also be

available for print production, either before or after insertion of format

codes. Compositors tapes and CD-ROM are likely to be particularly relevant

to this application.

Although there is a trend towards Desk Top Publishing (DTP) in the PC

environment, and the top-end packages offer a high level of sophistication,

it may be preferable to incorporate a professional type-setting system.

This would enable RNIB to handle most of its printing in-house e.g. forms,

headed stationery etc. with the appropriate cost savings.

2.6. Archiving

It should become practice to commit to archive various forms

of the data e.g. clean text, translated but unformatted braille or Moon,

finished data. For this purpose a high density archive medium should be

considered e.g. WORM technology CDs. The fast access, security of data and

compactness are benefits when compared to magnetic tape.

A. regular regime of backing-up data onto streamer tape or disks will need

to become part of the system operation.

2.7. Networking

The 30+ work-stations are designed to function as stand-alone production

units able to work in any medium and support any output device. For reasons

of economy and efficiency it is preferable to allow a sharing of resources.

To this end it is planned to connect the work-stations and the various

input and output peripherals.

Clusters of 4 to 6 work-stations will probably be preferred, in order to

permit more effective supervisory control.

The proposed work practice will involve downloading the particular document

from the central file-store to the hard disk on the work-station from where

it can be edited or translated without imposing any processing overhead on

the file-server. On finishing a work session, or a job, the file will be

transferred back to the file-server from where it can backed-up or archived

centrally. Embossers, printers and plate-makers can be attached to work-

stations around the system or to dedicated PCs on the network.

If commercial typesetting systems are to be integrated with the other work-

stations on the network they may influence the choice of network topology.

3. Evaluation

3.1. Bespoke Option

A specification for a custom designed and built work-station to emulate the

very popular QVEC editing stations was commissioned but was not adopted,

partly on grounds of cost and partly as a result of disappointing earlier

experiences with BCTs, ie the potential problems of on-going support from a

small specialist company. There did not appear to be anything suitable on

the market, particularly from the point of view of braille displays and

suitable keyboards. It was at this point that it was decided to forgo the

luxury of a QVEC-style full screen display and compromise on a PC based

system. Exploration of the potential for a bespoke software solution led to

a similar conclusion re long term development and support, and also the

time scales involved seemed unattractive.

3.2. Software

The objective then became to use a standard package to cover all aspects of

input and editing in any of the media involved. Evaluation of software

against a set of criteria produced a short-list of options:

1. A text editing package from dolphin systems for the disabled which had

an excellent braille display but was not designed for page based

applications and as such had short-comings for the type of work involved

2. A word processor called ChiWriter designed for scientific applications,

which has a font designer built-in. This piece of software also had its

limitations, a serious one was the maximum of 100k bytes for a data file,

this limits braille files to approximately 90 pages, well short of the 130

or so pages of the average volume. Headlines at the top of each page are

not handled satisfactorily either in Chiwriter as there is not the

"running-head" feature found in most commercial wordprocessors.

3. A system based upon WordPerfect with an add-in braille font. A number of

font add-ins were investigated along with several of the leading commercial

word-processing software packages. At the time WordPerfect version 4.2 was

being used elsewhere within RNIB as the standard wp package.

The WordPerfect option appeared to offer the greatest potential for

satisfying the criteria, for text processing in each of the media required.

3.3. Hardware

It had already become apparent that successive generations of software were

demanding ever more powerful hardware to support them. As a result it had

been decided that Intel 80286 based micro-computers would be the standard

machine for general use. Although WordPerfect v4.2 will run happily on an

8086 level micro it is a little slow, v5.0 is rather hampered by the limits

imposed at this level of computer power. As further development proved,

even a reasonably fast and enhanced 80286 machine is at full stretch to

handle to requirements that the braille aspects of the work-station impose.

For the type-setting aspects of large print production it was decided to

step up a level of micro-processor power and employ computers based on

80386SX chips.

4. Development

4.1. Text

Text derived from KDEM files is edited in preparation for translation into

braille and, in future, Moon. Similarly, texts from other sources can be

prepared for further production processes. WordPerfect has the facility for

accepting ASCII files, and a variety of options are available for

converting files from other sources into WordPerfect formats. The powerful

macros facilities available make editing reasonably straight-forward.

4.2. Moon

The implementation of a Moon input system commenced with the development of

a Moon font for WordPerfect to drive a laser printer. The software was then

configured to make the font available for A4 and B4 landscape paper. Text

was able to be keyed in using the appropriate characters to represent the

contractions and allowing the wp software to handle the pagination. Moon

does not have headlines as braille does, but pages are numbered. At this

stage of development a number of page numbering schemes were tried out:

having the page number on a line on its own, but this was wasteful of paper

in a bulky medium; using a prepaginated columnar set-up and importing the

Moon text into it; manually applying page numbers to the finished pages

before final embossing. None of these was totally satisfactory and it is

only recently that a facility for positioning the page number at the right

hand end of the first line of text has been implemented. The Moon text

input and editing scheme was originally developed in WordPerfect v4.2 and

only upgraded to v5.0 when it was integrated with the braille system. At

this time a new version of the Moon font was developed to improve the

quality of the printed output.

From the laser printer output of the Moon pages of text a Minolta copying

system using "swell paper" was originally used to produce the final

embossed output. Because of the cost of "swell paper" production of Moon by

this method was limited. A photographic system is now used to produce

polymer masters from which pages can be pressed in a similar way to braille

pages. As far as the wp aspect is concerned there are no real differences

between Moon and standard text other than the font involved.

The image-setter supports the Postscript printer language and both braille

and Moon Postscript fonts have been designed in-house. Moon is now being

produced in WordPerfect using the PostScript Moon font and output to the

image-setter, this now accounts for about 80% of the image-setter's

capacity.

4.3. Braille

Development of the WordPerfect based braille system was delayed by several

months during the transition from WordPerfect v4.2 to v5.0. This was

principally due to a delay in development of the font designer FONTMAX in a

version for the new release of WordPerfect. The improvements and

enhancements available with v5.0 had the potential to provide a much more

sophisticated system and so the delay was endured. One of the major

advances offered by WordPerfect v5.0 was the facility to preview the page

as it would look on printing, including headlines and page numbers in situ,

and FONTMAX provided this in a braille representation. This is essential as

there is no headline shown on the text editing screen.

One of the drawbacks of both the ChiWriter and WordPerfect braille systems

is that they are not able to use the 40 column mode of the PC, so the

braille characters only use half of the screen and are not as large as

would be liked. On the other hand, with a colour screen, WordPerfect offers

the option to select from a palette of colours to suit your own preference

for background and foreground. Some amazing combinations are possible!

A chording program, "SIX" from dolphin systems for the disabled was

originally used to reflect the usage of "Perkins-style" keyboards to

produce the braille cells. This program only applies when CAPSLOCK is

active, so allowing responses to prompts from the software to be entered in

either braille characters or print.

The first evaluation version of the braille aspect of the work-station

employed macros assigned to ALT-key combinations to provide shortcuts or

braille-specific facilities. This approach was refined as a result of user

feed-back and was greatly improved by using the WordPerfect facility to re-

map the keyboard and so assign new functions where appropriate to

WordPerfect use of function keys. A list of the function key assignments is

included as an appendix to this paper.

The system as described is demanding of computer resources. WordPerfect

alone requires 384k, including FONTMAX takes the memory requirements upto

400k. The addition of the "chording" means that 425k of the 577k available

after DOS has been loaded is accounted for and only about 150k is available

for data. This may seem adequate for most purposes, but the later addition

of network software will reduce that figure by a further 70k. On top of

this each braille character takes up 4 bytes of memory, as it consists of

both a start and end byte around a character set value and the actual

character value within that character set. In this arrangement the standard

130 page volume, which takes up perhaps 100k in ASCII characters expands to

maybe 350k in braille dot format. Using the hard disk for overflow files,

as WordPerfect does by default, results in frequent disk transfers of

blocks of data and also produced corruptions of data on large files. The

addition of 512k of extended memory to each workstation and assigning it as

a RAMDRIVE for use by WordPerfect produced a major improvement.

Unfortunately, it did not completely solve the problem. On occasion, for no

apparent reason, a data corruption will occur which results in out-of-range

characters being generated. Attempts to delete one of these characters

frequently results in a proliferation of invalid characters and tracts of

braille are no longer accessible, in effect lost. This is obviously not

good for users' morale, but investigations are underway to identify the

cause and determine a solution. Apart from this serious problem the system

is generally well liked by users.

4.4. Large Print

Although a Desk-Top-Publishing package could produce the type fonts etc

that are required for large print readers, there was a requirement to drive

an imagesetter to produce the quality of output necessary for the

anticipated printing demands. A number of type-setting systems were

evaluated, based on all manner of computer platforms. A decision was made

to install a Hell-Zenotron image-setter and to front-end it with software

called 3B2, PC DOS based, readily networkable and compatible with

WordPerfect.

4.5. Translation Software

A number of braille translation systems were evaluated to determine the

most suitable for RNIB's requirements. Some of the features of "house

style" which were required were not available, although modifications were

offered by suppliers. In view of the anticipated changes to the braille

codes likely over the coming few years it was decided that an inhouse

solution may be appropriate. At the same time as the evaluation was taking

place a project to develop a music translation system was also underway. As

text to braille translation was to form an essential part of the music

system, it was decided to develop that aspect as a priority to provide the

required software. Using the DOTSYS software from the mini-computer as a

starting point a new version of the translator was written in the "C"

language for the PC. As well as music it is planned to add mathematics and

science codes t the text and computer braille currently available.

A translator for text-to-Moon has recently been developed and is currently

undergoing testing. This translator is compatible with PC-DOTSYS in that

the same formatting codes are used.

Also under development is a project to develop a "front end" to facilitate

easier conversion of text to braille and, in due course, Moon. This will

provide for recognition of text formatting information in its native mode

in a number of leading word-processing packages.

4.6. Bespoke Work-station Features

Apart from fonts supported and page formats the Moon aspect of the work-

station does not require any adaptation from the standard WordPerfect

package, nor does the print application. The braille implementation

represents the major adaptation and a summary of the function key

assignments is presented as an appendix to this paper. An explanation of

some of the features should help to clarify the rationale of the system.

Many of the standard features available in WordPerfect are not applicable

to braille, such as Bold and Underline etc, so where appropriate the

function key assignments were changed to braille-specific or useful

functions. In several instances the opportunity to shortcut several levels

of WordPerfect's sub-menus was taken, eg defining headers involves 5 levels

of menu choice - a

function key now takes the user straight to the header input screen. The

same function key in conjunction with the <Shift> key offers the most

recent header for editing.

A major problem with the FONTMAX add-in to WordPerfect is that is does not

allow Search and Replace in the font selected. To counter this it was

necessary to provide an alternative set of functions to perform these

operations. In order to facilitate Search and Replace it was necessary to

use the second document feature and assign the string to be searched for to

an internal variable which could then be used as a parameter to

WordPerfect's own Search function. This was extended to assign a braille

string to a second variable to facilitate the Replace feature. Although not

ideal it does work! A side-effect of having to develop enhanced functions

is that sometimes they could prove to be fragile with the result that a

number of alternative keyboards were designed which limited options to the

user in order to minimise the risk of crashing the system. One of these

keyboards is specific to the on-line "help" system provided - again using

the second document feature, but limiting the user to browsing the screens

of information and exiting. Even this is not totally fail-safe so there is

an option on every bespoke alternative keyboard to return to the standard

default WordPerfect keyboard from whence a restoration of normal service

can be achieved.

Some function keys are dedicated to simplifying implementation of RNIB/BAUK

"house style" features, such as the 3-space paragraph used in magazines.

This is achieved by using the "hard-space" character which prevents word-

wrap between words. In effect 2 "hard-spaces" are linked to the word that

follows, thus producing the cell-3 start to a standard paragraph if the

whole string is carried down to the following line.

Another of the "house style" features involves the use of print-page

numbers in the braille text. Standard practice is to include the current

print-page number at the left of the headline and whenever the print-page

number changes an indicator, consisting of a dot 5, dots 2 and 5 and then

the number, should appear centred on a line on their own at the precise

point of the print-page turn. By the use of a function key the user is

prompted to enter the new print-page number and then both the headline

element and the in-text element are generated automatically.

As mentioned elsewhere, the headline feature in WordPerfect is widely

appreciated but it does have one quirk, which does not apply in braille,

which is that a blank line is generated to immediately follow the headline.

This blank line is removed via a tidy-up program later in the process, but

it does mean that the line-count is always 1 line out on the editing

screen. Another function of the tidy-up program is to convert the page

numbers assigned by WordPerfect, as it paginates, into the equivalent

braille numbers.

Because of the way in which the WordPerfect/FONTMAX combination stores the

braille characters it is necessary to convert them into ASCII before they

can be used by braille devices. WordPerfect provides an ASCII printer

called Dos Text Printer which combines the page information with ASCII

text. Editing of the character tables for this printer driver facilitated

the conversion of the complex internal representation of the braille into

ASCII values. By printing the file to disk and then running the tidy-up

program outside of WordPerfect a standard ASCII braille file is produced.

The printing to disk would normally involve delving through 4 levels of

menus to achieve but is available on the function key that WordPerfect

normally assigns to "Print".

If dot representation on screen is not required the same function key

features are available to users working with ASCII braille characters, the

original WordPerfect Search and Replace functions are then available. If

"chording" is not required ASCII input of grade II braille using the normal

keyboard can present the characters in dots or not as required. The ASCII

display can be used by visually handicapped people via a tactile display -

speech output is not ideal for ASCII braille.

The facility to take an ASCII braille file and convert it into a

WordPerfect/FONTMAX file has also been incorporated. Not a lot of use has

been made of this facility as yet, probably because it takes more than an

hour to convert a standard volume, including handling extraction and

encoding of the headlines. This option has been used where extensive

editing has been required on a file for which only the ASCII version is

available, as users tend to prefer to edit in braille dot format. Files

archived from the old mini-computer system also come into this category.

5. Future Enhancements

The serious problem of system overload generating out-of-range characters

is the top priority. In an attempt to cut down the number of programs

involved the replacement of FONTMAX is being evaluated. WordPerfect offers

the facility to redefine the screen character map and it has been possible

to produce the appropriate braille display mapped onto a new keyboard.

Still to be achieved is a page-preview in dots and the chording facility

for six-key input needs to be modified to accept the new set of key codes.

6. Archiving

Because of the non-volatile nature of completed texts it was decided to

implement a WORM drive for long-term archiving. The 400mb capacity per disk

is great improvement over the 800 bpi tape that was used previously. The

direct access ability of the WORM is another advantage over magnetic tape,

even though access times are slow compared to disk drives. Magnetic

streamer tape is used to back-up the hard disk on the network file-server.

7. Network

A Novell Netware 386 system was installed in April 1990. The features of

the network are:

- To enable upto 50 users of the transcription workstations involved in

producing braille, Moon and large print to have access to shared resources,

primarily braille embossers and laser printers

- To have centralised filing of data and a common library of macros and

menus for the WordPerfect based system

- To facilitate management of data files for archiving and backup centrally

- To provide a production monitoring system using a PC database, accessible

to upto 20 users in Production Control, Production management and Marketing

- To allow upto 10 users in the Editorial and Proofreading departments t

have access to aspects of the system

After a few teething problems the network is now functioning to

specification.

Each transcription work-station has its own copy of WordPerfect v5.0 but

uses the file server for file storage and as the source of all macros and

bespoke keyboard layouts. This ensures that everyone is using the same

version of the macros etc. It was not possible to implement v5.1 of

WordPerfect as FONTMAX was not compatible with it. Version 5.0 was no

longer being supplied when the network was implemented so a fully networked

WordPerfect system is not currently available.

8. Summary

The objectives have, for the most part, been achieved,although there is

still some work to be done in resolving the intermittent out-of-range

character problem.

I would like to acknowledge the help of my colleagues, and also thank them

for their patience!

9. Appendix

Braille Work-station redefined Functions

<F1> cancels most operations. Do not use with the search and replace

functions. To abort search or replace enter just <CR> at each prompt.

<Shift F1> facilitates screen colour customisation. Press <CR> when

finished. Serious damage to your, and other people's, sanity may result

from inconsiderate use.

<F2> to use the search facility when in braille display mode. <CR> is the

terminator.

<Shift><F2> to search backwards through the file. <CR> is the terminator.

<ALT><F2> to enable search and replace in braille display mode. <CR> is the

terminator.

<F3> help screen.

<Shift><F3> to change the braille page number. Enter the number in regular

text. It will be converted to braille by the tidy-up program.

<Alt><F3> reveal codes. <F11> does the same.

<Ctrl><F3> to refresh the screen (if major reformatting causes temporary

problems).

<F4> positions the margin 2 cells in and all run-overs are at that cell

until the next <CR>.

<Shift><F4> Produces a 2 cell indented run-over.

<Alt><F4> turn BLOCK on. <F12> does the same.

<Ctrl><F4> will move the marked Block. Move the cursor to position you want

text to be moved to and press <CR>.

<F5> Lists files in the default directory

<Shift><F5> for inputing print page numbers at the top left of the page.

The dot 5, middle C is also centered on the current line with the braille

number that you enter.

<Alt><F5> edit headline element of print page number. The cursor must be

after the defined headline.

<Ctrl><F5> will copy the marked Block. Move the cursor to position you want

text to be copied to and press <CR>.

<F6> allows for 12 centered characters - enter the character you want

centered. Terminate with <CR>.

<Shift><F6> centre characters until <CR> is pressed.

<Alt><F6> Flush right - terminate with <CR>.

<F7> Exit.

<Shift><F7> copies the current screen file to a file on the drive entered

as a response to the prompt, with the file name you supply at the second

prompt. Do not give the file an extension, a default extension of .BL is

generated automatically.

<Alt><F7> displays the printer control screen. If after issuing the print

function <Shift><F7> nothing appears to be happening, check the print

status and cancel the job or ask for help.

<F8> for inputing a centered headline - press <F7>

to exit!

<Shift><F8> for editing a headline. The cursor must be after the defined

headline. Press <F7> to exit.

<F9> gives three centered asterisks.

<Shift><F9> gives a three space paragraph.

<F10> save file: it is recommended that you save before "printing", just in

case anything goes wrong!

<Shift><F10> retrieve a file. The default directory is on the file-server.

<F11> reveal codes. The same as <Alt><F3>.

<Shift><F11> to preview page.

<F12> turn BLOCK on. The same as <Alt><F4>.