A university-based R&D program for technical aids

P. Mayer, N. Winkler, M. Busboom, W.L. Zagler

Summary

FORTec is a university-based working group dedicated to research and

development in the field of technical aids for the disabled. During the

last few years, several assistive devices for the blind and visually

impaired have been developed. In this paper, three projects of particular

scientific and practical relevance will be described:

- NotaPhon  - a braille input / speech output note-taker and communication

device for the blind.

- ColorTest - a speech output colour probe for the blind and colour blind.

- I.R.I.S. - an infrared operated information and orientation system for

the blind and visually impaired.

1. Introduction: FORTec and its R&D Program

The "Working Group on Rehabilitation Engineering" (FORTec) was established

in 1986 at the Institute of Electronics, Technical University of Vienna.

The group consists of 8 staff members and several students involved in

seminars and Master of Electrical Engineering Theses. FORTec projects are

financed entirely through government and private grants.

FORTec is the R&D arm of a three-pronged approach to promote the

development of new technical aids. FORTec is supported by ARTec (a non-

profit association for the promotion of rehabilitation engineering)

conducting all the necessary public relations by staying in touch with both

the disabled and public agencies. In order to ensure the production and

distribution of our ideas and products, FORTec saw a need to found its own

company called CareTec.

2. NotaPhon - A pocket-sized speech output wordprocessor

Blind people are often constrained by reading and writing equipment which

is bulky and difficult to transport. Although most modern electronic

braille writing equipment is portable when compared with mechanical braille

writing devices, there is an urgent need for further miniaturization. To

this end, FORTec developed NotaPhon, a miniature, battery-powered speech

output note-taker for the blind, initially for German, with the future

option of providing other languages.

The basic research for NotaPhon was supported by the Austrian Ministry of

Science. In the spring and summer of 1990, several NotaPhon units were

fieldtested by blind individuals. Production and distribution of the device

will be carried out by CareTec Corporation. Final production and marketing

are being funded with FFF and WWFF grants.

2.1  The Concept of NotaPhon

The basic idea was to develop a new tool for the blind which would meet

most of their daily communication requirements: Note-taking, calculating,

checking time, appointment making, address and phone number referencing and

interfacing with computers and other data equipment. Additional features we

planned were: Ease of operation, minimum size and weight, battery power for

6 to 8 hours without recharging and flexibility with respect to upgrades

and accessories.

2.2  NotaPhon Input - The Keyboard

NotaPhon has a 10-key keyboard, eight of which are used for braille input,

one control key and a spacebar. After extensive ergonomic studies and

testing, we opted for a double-slanted keyboard design: The keyboard itself

is slanted by some 5 degrees. The right- and left-hand keys respectively

have a 10 degree outward orientation thereby allowing for an uncramped and

natural brailling posture. The use of silicon-rubber contact elements

renders the keyboard virtually silent and gives the keys a pleasant snap-

action. A special mechanism in the key avoids jamming and provides the same

key pressure no matter where the key is depressed.

2.3  NotaPhon Output - The Speech Synthesizer

At present, NotaPhon uses the SSI 263A chip for full text german speech

synthesis. Other languages will later be available upon demand. The way in

which NotaPhon speaks to the user can be widely varied. For example, it is

not only possible to vary the pitch, rate and volume, but text parameters

such as audible punctuation, word-spelling and capitalization can be easily

detected.

2.4  RAM Memory vs. Floppy Disk Drive

When we decided to keep NotaPhon as light and small as possible, it was

obvious that a built-in floppy disk drive would be extremely disadvanta-

geous. On the other hand, to use only a battery buffered RAM of fixed size

would compel the user to constantly upload his or her work to other

devices. To avoid this, NotaPhon was equipped with creditcard-sized, user

interchangeable RAM cards which are currently available in a range from 16

up to 512 kB in size (units up to 4 MB to come later). The RAM cards are

battery-powered and will preserve data for up to five years. This

technology offers several of advantages: Compactness and lightweight, no

mechanical parts, user selectable memory size, compartmentalisation of

files in categories on individual cards and portability of the storage

medium.

2.5  NotaPhon Interfaces

NotaPhon is equipped with a standard RS-232 interface (9-pin connector) for

serial i/o operation. Parallel printers can be connected via a serial-to-

parallel interface module. A standard telephone connector in conjunction

with an internal modem are planned for future versions of NotaPhon.

2.6  NotaPhon User Shell

In order to maximize efficiency and user friendliness, a special user shell

was designed. Mnemonic in structure and keyboard hierarchy, this shell

allows the experienced user to circumvent often long menues and quickly

execute any desired function. Yet if required, the novice is literally

"led" to a specific command or function. Help for all commands and

functions is always available. Within the user shell, the amount of speech

(user prompting) and type of speech (full text or individual characters)

can be instantaneously set or varied. The user shell can be fine tuned to

ensure its operation with all planned NotaPhon languages.

2.7  NotaPhon Software Features

NotaPhon's editor is comfortable to use and allows the user to manipulate

text in much the same manner as a PC user. Single characters or larger text

units can be inserted, deleted, moved or copied. NotaPhon in general, and

its editor, in particular, are very ergonomic in that they are built around

the philosophy of combining the comfort and familiarity of braille with

speech output.

Most braille literate people use a form of contracted braille known as

Grade II braille which speeds up the reading and writing processes. The

disadvantage of this code - from the standpoint of speech output devices -

is that it cannot be directly used to generate synthetic speech. To solve

this problem, FORTec developed and installed "WineTU", a German language

backtranslator from Grade II braille to ASCII. It is now possible for

braille users to rapidly enter text in German Grade II braille, even if

print output is required.

Not only is NotaPhon valuable as a note-taker; the device has many

additional features:

- NotaPhon can be used as a talking terminal;

- NotaPhon will offer PC screen review facilities;

- Talking pocket calculator (basic and scientific);

- Talking clock, stopwatch, alarm clock and appointment calendar;

- Database for names, addresses and telephone numbers.

Some planned future upgrades:

- Built-in modem

- External floppy disk drive

- Forth interpreter

2.8  Technical Specifications

NotaPhon consists of two printed circuit boards. One of which is for the

keyboard and all analog-audio components (speaker, earphone-jack and volume

control). The other PCB contains the Hitachi 6303 single chip processor,

126 kB EPROM, 64 kB RAM, the connector for the RAM card, the real-time

clock, the SSI 263A speech synthesizer, interface and charging circuitry

and the back-up battery.

Size: 22 x 10 x 3 cm = 8.5 x 4 x 1.2 in.

Weight: ca. 500g = 16 oz.

Battery type: 6 AA cells (alkaline or NiCd)

Interface: 9-pin RS232 connector (male)

Picture 1: NotaPhon

3.  ColorTest - A Colour Detector with Speech Output

When other factors permit, blind people can live very independently.

Independence, however, is often decreased through an inability to

recognize, for example, the colour of two similarly shaped packages or the

colour coordination of clothes. The goal of the project "ColorTest" was the

development of a hand-held probe which would communicate the colour of an

object to a blind person in synthetic speech. Several battery-powered

prototypes have been built and successfully tested. This project was

sponsored through funds provided by the Austrian Ministry of Science.

Attempts will be made to distribute ColorTest in all important languages

and to market it through CareTec corporation.

3.1  Colour Measurement and Speech Synthesis

Colour measurement is accomplished by focusing white light onto the object

in question and measuring the colour components (red/green/blue) of the

reflected light with an integrated three-colour sensor. The unit can

distinguish between 50 different colours and shades.

ColorTest uses digitized human speech to convey information to the user.

Prerecorded speech has three distinct advantages over fully synthetic

speech. Firstly, it is much cheaper to produce since a human need merely

say each colour once into a recording unit. Secondly, any language can be

quickly implemented. Thirdly, since the speech sounds human, it is very

easily understood.

3.2  ColorTest - Features and Operation

Not only does ColorTest convey the colour that it "sees" to a user; it

provides him/her with other pertinent information as well. For example, it

is possible for the user to acquire a breakdown of colour components in

terms of their percentages within the reflected light. Thus, it is possible

for accurate comparisons of similar colour shades to be made.  Colour

measurement occurs at the push of a button. However, the measurements are

first announced when the button is released, thereby enabling the user to

place the unit near to his/her ear to compensate for noisy environments. If

necessary, all spoken information can be repeated.

In all probability, ColorTest will inaccurately measure the colour of

glossy surfaces (e.g. objects behind glass), transparent objects or the

colour of liquids. Furthermore, measurements of both detailed and ragged

surfaces are problematic.

3.3  Brief Technical Description

ColorTest uses the Hitachi 6303 single chip processor, 64 kB EPROM and a

parameter speech synthesizer.

Size: 150 x 40 x 20 mm = 6 x 1.5 x 0.8 in.

Weight: ca. 125 g = 2.3 oz.

Battery type: Standard 9 V rectangular battery (alkaline or NiCd)

Picture 2: ColorTest

4.  I.R.I.S. - An Infrared Information and Orientation System

During the past few years, many activities were undertaken to enable the

disabled to be more independent. The public became more conscious of

disabilities and began removing barriers. The mobility of the blind and

partially sighted has been improved, but few attempts have been made to

narrow the information gap which still remains.

4.1  Problem Areas in Mobility and Navigation

The information gap persists because a great deal of pertinent navigational

information cannot be easily conveyed through tactile or acoustic means.

Some examples:

- Traffic lights: "Visualization" of traffic light status would enable

blind persons to increase their mobility.

- Public transport: If more than one bus or tram line frequents a stop,  a

blind person cannot readily determine which line is presently at the stop.

- Orientation in public buildings: In elevators or large buildings a blind

person often has difficulty in determining which floor he/she is presently

on.

- Using electronic equipment: An increasing number of modern electronic

devices (public telephones, automatic bank tellers etc.) are equipped with

optical displays which cannot be read by the blind.

4.2  The I.R.I.S. Solution

I.R.I.S. will help fill this information gap by combining two well-proved

techniques:

- Transfer of digital information by infrared signals (widespread in the

remote control of HiFi and TV sets).

- Conversion of digital signals to synthetic speech (many speech

synthesizers are readily available).

4.3  The I.R.I.S. Transmitter

The I.R.I.S. transmitter should be simple and inexpensive so that it can be

installed in a wide variety of devices. Depending on the area of

application, different types of transmitters would be implemented:

- Transmitter Type I, the simplest, would only send out a variety of AM

tones. This would enable the receiver to be used as a homing device for

locating specific points. Type I transmitters, therefore,  could be used

effectively to indicate location and status of traffic lights. The

transmitter would have a range of up to 20 m (60 ft.).

- Transmitter Type II simply converts ASCII information to digital infrared

signals (e.g. applicable for bank tellers). This type would use pulsecode

modulation and have a range of up to 30 m (90 ft.).

- Type III (same technical specifications as type II) would be a more

intelligent transmitter, outfitted with a memory containing preprogrammed

texts. Selection of the words or sentences to be transmitted is achieved

through switches or electronic signals. A repertoire of texts, however, is

stored in the receiver and can be activated simply by transmitting a

digital code. Such a transmitter would be installed on public

transportation vehicles, elevators, etc.

Picture 3: I.R.I.S. transmitter mounted on traffic light

4.4  The I.R.I.S. Receiver

The blind user would carry a pocket-size receiver or wear a special type of

eyeglasses. The received infrared pulses are decoded and converted to

audible tones or spoken messages by the built-in speech synthesizer and

sent to an earphone. Two separated IR receiver diodes permit the user to

locate the transmitter by means of the stereophonic audible impression

produced by different signal levels of a pilot tone and/or the speech

output on each channel.

In order to realize a language-independent means of information

transmission, two modes of signal transmission are proposed:

- Every receiver would be equipped with a comprehensive set of "pictogram-

like" words and phrases in the language of the user. Each of these elements

is assigned a digital code which is activated by the transmitter. By

properly combining such code sequences, messages in the language of the

user are assembled. Proper names are produced by transmitting codes

corresponding to the international phonetic alphabet which are reassembled

phonemically in the receiver.

- Multilingual transmission of information is achieved by using different

IR channels or by time-sliced coding (e.g. for museum or tourist

information guides).

5. Conclusion

Although our experience has revealed that universities are ideally suited

for R&D in the field of technical aids for the disabled, they do not always

provide the necessary infrastructure for the marketing and distribution of

their products and ideas. Hence, we have attempted to illustrate from the

three projects described above that an external entity (i.e. a partner

company) cannot only enhance the chances for a product's survival but pump

new vitality and insight into a university's research approach by

monitoring the pulse of a target group's needs.