Electronic Newspaper System for Visually Handicapped People.

O. Sueda, K. Kato and Y. Oosaki.

1. Introduction

At present, the quantity of information, which can be obtained in the form

of braille or speech by a visually handicapped person in Japan, is not

satisfactory.   In case of the important information source "newspaper",

many services such as reading service, database system for personal

computer networks and so on have been prepared.   Although a visually

handicapped person can receive the above services, his situation is

different from that of a sighted person in timing, quality and quantity.

Research on the broadcast system for newspapers using FM radio frequency

has been done in Sweden1).  We think this broadcast system is useful as a

new method of publication service for visually handicapped people.

The target of our research is to develop a Japanese data broadcast system

for visually handicapped people in Japan.  In this system, the cable

network for music broadcast of Osaka Yuusen Housou Co. Ltd., which has 440

channels, is used as the means of transmitting data.

2. Data receiver for Japanese

In our electronic newspaper system, a partially sighted person is also

considered as a user, the same as a blind person.  A receiver has been

designed which has the functions of braille display, synthetic speech and

expanded display of the text on a monitor television.

In Japan, ordinary texts are a mixture of letters from the English

alphabet, Greek letters and symbols of mathematics and chemistry, the two

Japanese phonetic alphabets (hiragana and katakana), and several thousand

Chinese characters, or Kanji.   On the other hand, the textbooks used at

schools for the blind are written in the Japanese phonetic alphabets

without Kanji characters, which are the main elements of Japanese in

ordinary books.  In a Japanese sentence, many compound words of more than

two Kanji characters are used and there are many phrases with the same

pronunciation, although their meanings are different.   Therefore, simply

hearing a Japanese sentence sometimes brings about misunderstandings and

reading a Japanese sentence written in the phonetic alphabet brings about

more misunderstandings because of the lack of accents and/or rhythms.

The Kanji braille codes were invented by T.Kawakami in 1969, and developed

for educational purposes by Mr. and Mrs. Kawakami.   The expanding braille

code system to express assorted letters, symbols and graphic patterns was

developed by O. Sueda (1985)2), including Kanji braille.   In the new

braille code system, free combination of various mathematical, physical and

chemical symbols, and music scores into normal Japanese sentences, as well

as writing computer programs, is possible.

The data receiver for Japanese sentences should prepare the character

display devices, or device, for the Kanji characters in ink-print and/or in

braille.  The synthetic speech device is also useful as a supplementary

device

3. Problems of transmitting newspaper data

There are three big problems in the transmission of newspaper data.  The

first is how to protect the copyright; the second is how to increase the

reliability of received data against noise; the third is how to edit the

news to maintain readability in baille.

In Japan, print translated into braille from ink-print is free from

copyright.   The number of receiver will be more limited than that of the

system developed in Sweden because of the use of a closed cable system

compared with FM radio broadcast.   However, in case of the electronic

braille newspaper system, we transmit the data by usual computer code which

is readable by the sighted person on a monitor television.   In order to

solve the first problem, the receiver is numbered by the hardware, ID

number, which is used to open the gate according to the electronic key

which is transmitted precedently for the main data to the registered

receiver, and the data is ciphered/deciphered for transmitting/receiving by

the electronic key which is changed daily.

How to transmit data reliably is the general problem of communication.   In

our cable system for music, each channel has several KHz band widths and

for digital signal, the reasonable speed of transmission is about 4,800

Bauds.   The morning newspaper has about 300 kilo bytes of data and about

ten minutes will be neccessary to transmit all the data.   The data is cut

into blocks of 112 bytes and transmitted with a block check code.   The

whole of the data is transmitted three times.   The block of data which has

errors is omitted and filled by null data at the first transmission, and

that block will be filled by correct data at the second or the third

transmission.

A weather forecast and a table of TV programs are also very interesting for

visually handicapped people.   However, these items are not produced in

text format and it is very difficult to display them in braille or in

speaking.   An advertisement column is also very useful for handicapped

people.  However, its block copy is made by photograph and no text data in

newspaper file, although it is constructed just by characters.   A

financial column includes many tables as data and these tables are very

difficult to translate automatically into braille.   Some editings and some

rewriting of news is done at the braille printing house before

transmitting, in order to facilitate the automatic translation into braille

at the receiver side.

4. Hardware of the electronic braille newspaper system

An electronic newspaper publishing system is made up of a personal computer

system and a modem box for transmitting.   An electronic newspaper

receiving system is made up of a receiver for cable broadcast, a modem box

for receiving and a personal computer system for the visually handicapped

person with a braille keyboard, a braille display unit, a synthetic speech

unit and a braille printer.   The modem box for transmitting data has the

functions of ciphering of data according to the key code and producing of

quasi X-modem protocol.   The modem box for receiving data has the

functions of an electronic key for protection of copyright against non-

registered receivers and deciphering of data.   Each receiving modem box is

numbered by hardware, ID number, for controlling of opening/closing of the

deciphering circuit and data rearrangement function.

5. Software of the electronic braille newspaper system

The data of the morning newspaper is mailed at the braille printing house

of Nippon Light House early in the morning, at 2 or 3 AM.   At first, the

data is automatically cut into each news file with the header, which

containes attributes of the news such as column number, page number of

original ink-print and a number of news items.   A control data file is

produced every day which is transmitted precedently for the main data of

news, constructed by the electronic key data for cipher/decipher and

receiver control date according to the ID number of each receiver which is

registered for reading the newspaper.   The small amount of editing of news

is done manually by the professional staffs of braille translation at the

printing house.   After these arrangements are finished, the control data

file and main data files are sent three times to the transmitting modem box

connected to the transmitter of the special channel of cable broadcast for

visually handicapped people.

A receiver wake up according to the control data is received precedingly

for the news data, if it is the registered receiver.   The news data is

deciphered according to the decipher key data, and sent to the personal

computer by a block data with the RS-232C serial port.   On the personal

computer, blocks of data are gathered up and formed into a news file, and

it is saved in a floppy disk.   If an error occurred during the first

receiving of data, the errored block of data is omitted and filled by null.

At the second or the third period of receiving, the block of data, which

would be error free, is inserted into the old file.   The probability of

missing the same block three times is designed in a very small value,

p=(0.0088)3=0.00000068, and the total number of blocks are about three

thousand.

The user of the electronic newspaper system can read the news data in the

morning with the reading program in braille display and in speaking and can

print it out in braille.   At first, the user selects the page and secondly

selects the news guided by the title and the abstract.

6. Conclusion

In Japan, the cable broadcast system covering almost all Japan is operated

by one big company, Osaka Yuusen Housou Co. Ltd., and one of four hundred

forty channnels is used for visually handicapped person to send the news

and to read the newspapers.  This situation is of benefit to the

development of an electronic newspaper system.  According to this research,

many problems are also revealed, such as the rewriting of tables, omitting

of the graphic news in advertisement columns, the use of special Kanji

characters out of the standard code table, mainly for the name of persons,

and so on.

This research has been conducted as a joint project with Naruto University

of Education and Nippon Light House by the fund from Mitsubishi Foundation,

1988.

The Asahi newspaper Co. Ltd. and Osaka Yuusen Housou Co. Ltd. have also

assisted this research.

7. References

1) H. Rubinstein : Radio-Distributed Digital Daily Newspaper for the Blind,

Proc. of 2nd Int. Conf. on Rehabilitation Eng., pp583-584, 1984.

2) O. Sueda : Eight-dot Braille Code and Automatic Braille Translation

Boards, Proc. of 5th Int. Workshop on Computerized Braille Production,

pp273-303, 1985.