PC-Voice : A Dutch text-to-speech aid for the visually handicapped.

Bert Van Coile, Bart No and Jean-Pierre Martens.

1. Introduction

In this paper we describe a speech output system called PC-VOICE.  The

device offers full text-to-speech capabilities for Dutch.  Connected to a

computer (e.g. a PC), it can serve as a reading device for the visually

handicapped.  Obviously, the speech synthesizer can also be a tremendous

aid for the vocally handicapped (Martens et al 1990).  However, this issue

is beyond the scope of this paper.

We briefly outline the principles underlying the text-to-speech synthesis

strategy, and indicate how we were able to implement everything in hard-

and software. Subjective tests have proved that the system generates

intelligible speech.

2. The text-to-speech strategy

The text-to-speech strategy as it is implemented in PC-VOICE embodies three

major parts : a linguistic, a phonetic and a speech synthesis part.

The linguistic part is responsible for the phonetic transcription of the

typewritten input text.  In addition it extracts syntactic, lexical and

semantic information from the input text.

The phonetic part employs the information made available by the linguistic

part to construct an acceptable prosodic pattern, and to create the correct

sequence of speech parameters to drive an artificial speech production

model.

The speech synthesis part incorporates the artificial speech production

model which is used to convert the speech parameters into an acoustical

signal.

In the following sections, we describe the linguistic and phonetic parts in

more detail.  For more details about the speech synthesis part we refer to

Van Coile and Martens (1989)

3. The linguistic section

The linguistic section of our present system deals mainly with the

conversion of the input text into its phonetic transcription.  Several

approaches may be used to perform this orthographic-to-phonetic

transcription.  The simplest solution is to use a large phonetic

dictionary.  However, in order to be useful, the number of dictionary

entries must be very large.  Even with an enormous amount of words stored,

the system will not be able to convert every text into its phonetic

representation.  This is partly due to the existence of generative rules

which describe how to create new words from existing words, prefixes and

suffixes.  Therefore, a more general approach is essential.  In our system,

the conversion of a text into its phonetic transcription is mainly done by

means of pronunciation rules.  The following pronunciation rule is

applicable to Dutch : 'At the end of a word, the letter [d] is pronounced

as a /t/'.  A set of such context dependent rules is called a linguistic

grammar.

When linguistic grammars are hard-coded into a computer program, it is

difficult to make changes.  In order to facilitate and speed up the

development and the testing of new grammars, we conceived a flexible text-

to-speech development tool called DEPES (Development Environment for

Pronunciation Expert Systems) (Van Coile, 1989).  DEPES offers a powerful

knowledge representation language which borrows from the well-known

formalism used in generative phonology.  The DEPES language was carefully

designed in order to combine high flexibility and ease-of-use.  DEPES also

provides several development utilities such as a compiler, a linker, a

debugger and a dictionary tool.  The DEPES development environment relieves

the user of as many details as possible and permits concentration on the

linguistic problem.  Even people without any programming experience can

develop and test their own linguistic rules.  The development tool was used

successfully for the development of the rule component of our text-to-

speech system.  This rule component mainly performs the following tasks :

- Deal with  special character sequences such as digit strings, date and

time indications, etc.

- Make a grapheme-to-phoneme conversion

- Realize the syllabification of the words

- Assign lexical stress

The system also includes some grammars with heuristics :

- To determine the major syntactic boundaries

- To introduce pauses correctly

- To determine the important words of a message

In order to perform all of these tasks, the rule component consists of

several linguistic grammars applicable to different items such as single

words, whole sentences, digit strings, etc.  The total number of linguistic

rules is of the order of 500.  The most complex and most important grammars

deal with the orthographic-to-phonetic transcription of single  words.  The

necessary linguistic knowledge was extracted from an orthographic-phonetic

dictionary containing the 10,000 most frequent words of Dutch.  The present

system uses about 300 rules to cover 92 % of these 10,000 words (96% if we

omit lexical stress  errors).  Those words which are mispronounced by the

rule-based system are stored in a dictionary of exceptions.  Apart from

this dictionary, the rule component also refers to two other small

dictionaries.  The first one contains Dutch abbreviations, the second one

comprises high frequency words together with part-of-speech information.

The complete system is able to perform the orthographic-phonetic

transcription with a high accuracy.  It has been estimated that less than 5

% of the words in an unrestricted text show some pronunciation defect.

This means that the phonetic transcription of less than one word in twenty

is not completely correct.

4. The phonetic section

The phonetic section of the system performs the segmental and prosodic

synthesis.  It includes modules :

- To create a correct durational pattern

- To synthesize an adequate intonation contour

- To create good spectral parameters

The system uses a durational model for Dutch that attributes a well

estimated duration to each phoneme.  In order to discover durational rules,

we analyzed the phone durations measured in a large amount of speech data.

For example, durational rules for continuous speech were determined on the

basis of a Dutch text with a total length of more than 8 minutes.  This

text was read by one female native speaker of Dutch.  The same speaker was

used during the development of the segmental synthesis part (see further).

Rules were developed to explain the bulk of durational variations observed

in the text.  These durational rules account for phenomena such as the

short/long opposition of Dutch vowels, the influence of prominence, word

final lengthening, prepausal lengthening, etc.  The durational rules in the

present version of our system account for 81 % of the total variance

observed in phoneme durations of a read text (Van Coile, 1987a).

A second important task of the phonetic section is to provide a correct

intonation contour for the message to be synthesized.  The system uses a

strategy based on the results of a study on Dutch intonation by 't Hart and

Cohen (1973).  The intonation contours are described in terms of

standardized rises and falls of pitch.  A limited number of rules specifies

how these elementary pitch movements can be combined to create an

intonation contour for a whole message.  These rules take into account the

number and the location of the dominant words and the major syntactic

boundaries.  This information is made available by the linguistic section

of the system. Given the description of the intonation contour in terms of

elementary pitch movements and given the durational information, the

fundamental frequency contour is calculated.

Finally, the segmental synthesis is performed.  Our system is based on the

segment concatenation technique : an inventory of small speech segments

taken from natural speech is used to compose any synthetic message.  An

interesting segment type in this respect is the diphone.  A diphone is a

small speech segment which starts and ends in the stationary parts of two

successive phonemes.  Consequently, the transition between phonemes is

preserved within the segment itself.  Therefore, the simple concatenation

of diphones taken from human speech already accounts for a lot of

coarticulation effects.  In our approach to speech synthesis we use an

inventory of diphones and triphones.  During the development of the system,

these segments were taken from isolated words spoken by a female speaker.

An LPC vocoder was used to analyse the words.  Subsequently, the segments

were extracted semi-automatically (Van Coile, 1987b) and stored in the

segment inventory.  During text-to-speech conversion, the inventory of

segments is used to perform the segmental synthesis.  This involves the

following steps :

- Subdividing the phonetic transcription into segments.

- Consulting the segment inventory and concatenating the speech parameters

of the different segments.

- Setting the estimated duration of each phoneme by stretching and

shrinking the time between successive parameter sets.

- Adding the calculated fundamental frequency contour.

The speech parameters are then used by the speech synthesizer to obtain the

acoustic waveform.

5. Hardware implementation

PC-VOICE is a device which receives its input text through a standard

serial link (RS232).  It already incorporates a loudspeaker which is

switched off as soon as a headphone or an external loudspeaker is connected

to the audio output jack. PC-VOICE measures 29 x 26 x 6 cm and weights 1.8

kg.

Text-to-speech synthesis involves different kinds of processing.  The

speech synthesis part mainly executes multiplications and additions, while

the linguistic and phonetic parts mostly perform logical inferences and

database retrieval operations.  Given the different nature of these two

kinds of processing, we have designed a system which incorporates a general

purpose microprocessor (Intel-8086) for the linguistic part, and a special

purpose signal processor (TMS320-10) for the signal processing part.  The

memory consists of 208 kbyte EPROM and 64 kbyte RAM.  The EPROMs contain

the speech segment inventory, data structures describing the linguistic and

phonetic knowledge, and several dictionaries. A personal dictionary (see

below) can be stored in the free space (24 kbyte) of the RAM memory.

6. Full text-to-speech capabilities with PC-voice

PC-VOICE is able to read aloud any ordinary spelled message it receives.

This includes the correct pronunciation of special character sequences such

as digit strings, telephone numbers, date and  time indications,

abbreviations, alphanumeric strings, etc.  PC-VOICE also offers the

possibility of changing voice characteristics (several male and female

voices), volume and speech rate.

In order to be useful as a reading machine for visually handicapped

persons, PC-VOICE must be used in connection with screen reading software

running on the host computer.  By means of this software, the text on the

screen can be read even during sessions with standard software products

such as text processing or data base programs.

PC-VOICE is delivered with PC-SCREEN, a free software product for IBM

compatibles which supports the basic screen reading operations.  However,

it should be mentioned that especially for IBM-PC compatibles, much more

advanced products have become commercially available (Meyers and Schreier,

1990).  Well known are SCREEN-READER from IBM, HALL from Apollo and FLIPPER

from Omnichron.

Also delivered with PC-VOICE is PC-DIC.  With this free software product

the user can create his own personal dictionary on his IBM compatible, and

add it to the pronunciation knowledge already available in PC-VOICE.  A

personal dictionary may include both ordinary spelled and phonetically

written translations.  Thus it can easily be used to expand uncommon

abbreviations, to define the pronunciation of foreign words, to correct

mispronunciations of words from the users own jargon, etc.

7. Voice intelligibility

A formal evaluation of the word intelligibility of the text-to-speech

synthesizer has been undertaken.  The listeners were divided into two

groups: untrained listeners who were unfamiliar with synthetic speech, and

trained listeners who were already familiar with PC-VOICE.  Ordinary Dutch

words were generated in isolation by the speech synthesizer, and the

listeners were asked to type the word they thought to perceive.  The tests

yielded an average word intelligibility of 83 % for untrained listeners,

and 96 % for the trained listeners.  The word intelligibility for untrained

listeners depends significantly on the number of syllables in the word.

For mono-syllabic words we measured a score of only 67.3 %, but for bi-

syllabic words the score already augmented to 83 %.  For multi-syllabic

words the recognition rate was larger than 90 %.  Since the words were

spoken in isolation, no semantic nor syntactic information was available to

the listener.  Therefore, the measured scores can be considered as lower

bounds for the actual word intelligibility.

8. Future developments

Thanks to the advance of microelectronics, more powerful electronic

components have become available.  Therefore, we are currently developing

new hardware which will be smaller and faster.  The second generation of

PC-VOICE will be half the size of the current generation and will respond

two to three times faster.  It will also be possible to plug in the

hardware into a portable IBM-AT compatible with free slots.

9. Conclusion

We have described PC-VOICE, a device offering full Dutch text-to-speech

capabilities.  With the proper screen reading software on a personal

computer, it can serve as a powerful reading device for the visually

handicapped.  Subjective tests have clearly demonstrated that the synthetic

speech is highly intelligible, especially to listeners who are familiar

with the device (more than 96 % word intelligibility).

10. Acknowledgements

This work was supported by the Belgian Ministery of Public Health, and by

the Van Goethem-Brichant Foundation (Belgium).

11. References

'tHart J. and Cohen R.  (1973).  "Intonation by Rule: a perceptual

question," Journal of Phonetics 1, 309-327.

Martens J.P., Van Coile B. and No B.  (1990).  "PC-STEM: een

spraakhulpmiddel voor spraak- en visueel gehandicapten,".  Communicatie

drieluik vol 2, no 1, 14-16 (in Dutch).

Meyers A. and Schreyer E.  (1990).  "An evaluation of speech access

programs,"  Journal of visual impairment and blindness, 26-38.

Van Coile B.  (1987a).  "A model of phoneme durations based on the analysis

of a read Dutch text,"  Procs. Europ. Conf. on Speech Technology 2, 232-

236.

Van Coile B.  (1987b).  "Computer aided segmentation of spoken words given

their orthographic representation,"  Procs. Europ. Conf. on Speech

Technology 1, 277-280.

Van Coile B.  (1989).  "The DEPES development system for text-to-speech

synthesis,"  Procs. IEEE Conf. on Acoust. Speech and Signal Proc. 89, 250-

253.

Van Coile B. and Martens J.P.  (1989).  "Dutch text-to-speech aids for the

vocally handicapped,"  Procs. Europ. Conf. Speech Comm. and Technology,

590-593.