Two mice for blind users of the pin-matrix device.

Gerhard Weber.

1. Introduction

Due to the lack of an adaptation of mouse-based user interfaces to the

needs of a blind user there has recently started to open up a gap between

those terminals usable by blind people and those which are unusable. While

this was only a problem for a minority of users who had sighted colleges

working with a Macintosh computer or who wanted to play a "video game" on a

PC, this starts now to concern more blind people. Especially with the

introduction of Presentation Manager and MS-Windows for the comercial users

of PCs, and X-Windows for the users of workstations in an University

environment there exists now a broad variety of software that is

inaccesible or at least only accessible after tedious data conversion

routines.

It is not that a blind engineer wants to take a job where work has to be

done by using a CAD system (even if his education would allow to do so),

but there are a growing number of jobs that rely to a very large extent on

working at a computer with a graphical contents of the terminal's display.

Since the information itself is stored in a computer we are currently

lacking of appropiate adaptations using non-visual output media. Mainly

terminals in control stations for large processing plants, or those for

railway lines are using colour as an easy method to overlay different

layers of a graphics. And in addition to that the more powerful word

processor packages also use colour and/or graphics to display the

information more densly.

Graphical user interfaces want to take advantage of what is expressed by

the proverb "a picture says a thousand words". In addition, the interaction

with visual objects on the basis of a mouse is not easily translated into a

verbal or braille representation. For example a common interaction method

is that of dragging an icon to other locations on the screen. It is unclear

how to explain this verbally with a speech synthesizer. And similarily a

one or two line braille display is no planar output display. But it are not

only restrictions on how to use a pointing device, it is sometimes just

faster to use a pointing device instead of a keyboard. For example, menus

are possibly read with a speech review program or with a braille display.

But the exploration of the planar organized texts does not include a

possibility to point to an option that is wanted. Instead it is usually

another step in the dialogue with the computer to bring a marker (cursor)

to an option of the menu and then select it with input from the keyboard.

On the other hand research into multi-media or even hypermedia promises to

have an impact in the near future into the design of user interfaces, e.g.

- Voice recordings are very useful in addition to written notes,

- Sounds may display more complex properties of visual objects, or

- Force feedback displays, that let the user feel objects of the user

interface.

But nonetheless it is not granted that these additional media - even if

they are based on accoustic or cutaneous sensations - are usable by blind

users.

There is a growing interest in combining pointing devices with non-visual

output media like speech [Hill88, Wihofsky90], sounds [Gaver89], or

Braille. Only recently manufactures have started to add a one-dimensional

pointing device to the well-known braille displays. With the help of these

switches resp. optical sensors that are placed above each braille module a

reader can force the cursor to move to the location where the finger is

currently pointing (this is commonly called "cursor routing").

In the following sections we broaden this notion of allowing the fingers to

specify additional input compared to simply type on a keyboard by

describing the more general concept of a touch-sensitive braille display

that has been realized with the touch sensitive pin-matrix device. Thereby

we open up not only an additional input channel but also allow for new

methods of interacting with graphical user interfaces.

2. The touch-sensitive pin-matrix device

In today's user interfaces, selecting an icon using a pointing device is

accompanied by immediate feedback between the hand's movements while

dragging the mouse and the movement of a marker on the display. Blind users

cannot get the necessary feedback to operate conventional pointing devices

while reading the contents of a display. A blind user would need an output

device which corresponds to the 2-dimensional nature of the screen and that

can provide the necessary feedback concerning pointing and reading to

establish a feedback loop.

Tactile braille displays like the Optacon [Bliss70] or the IB80

[Papenmeier] work as adapters for microcomputers. These devices exemplify

two different kinds of reading with fingers. These are what we call

- Passive reading and

- Active reading.

With passive reading, the user's finger remains stationary on a small

tactile display while the contents of the display moves under the finger.

In particulary, the contents of the display is continually refreshed

according to the direction in which reading occurs.

To enable the blind user to read actively the user's fingers - at the

present state of art in braille technology - must move over multiple

braille cells. The tactile surface should be large enough to represent in

detail the objects of the dialogue program.

Definitions :

- The finger's reading-region is the part of the finger's skin that is in

cutaneous contact with a surface.

- The process of reading is referred as being active when the finger's

reading-region is changed after a movement smaller than the width of the

reading-region of its hand has occurred. Otherwise it is referred to as

being passive.

Active reading is intimately related to active touch [Gibson62], which

involves additional nervous receptors thus providing more and richer

sensations [Gordon78] compared to passive touch. In the following sections,

we are able to develop types of interaction which build on this notion of

active reading.

Example: active or passive reading ?

The tactile displays proposed by Karshmer (see [Karshmer85] and for a

realization [NASA90]) and Anbary [Anbary81] use a single braille module

mounted on a sliding carriage (see Fig. 1). In all of these systems, the

coordinates of the carriage are projected onto a virtual screen, to get a

single braille character displayed. We want to call this type of display a

virtual display. The finger's reading-region does not change if the

carriage is not moved far enough to the next character's position.

Following the outline of a circle, for example, is therefore not possible

with these displays. Thus using this type of display, the user applies

passive reading.

The "Braille Mouse" proposed by DeWulf uses not only a single braille

display but two 6-dot braille cells that are mounted on a sliding carriage.

With the help of the second braille module, the "Braille mouse" allows to a

small degree active reading backward. There is to the knowledge of the

author currently no virtual display available that allows active reading.

In order to facilitate active reading, we use a large display at the

Department of Computer Science, referred to as the pin-matrix device (see

Fig. 2). This display, with more than 7000 pins, has been successfully used

to display videotex pages [Schweikhardt84] and geometric forms.

We implemented as a pointing device for the pin-matrix device a sensor

system based on detecting magnetic fields. It is attached to both of the

user's forefingers and allows active reading with the forefingers on the

display while the position of the detector is continually monitored. The

two recently built pin-matrix devices are equipped with an improved version

of this sensor system. Thus all of the three currently existing pin-matrix

devices are touch-sensitive.

With the touch-sensitive pin-matrix device, it is possible to design

interaction methods that are based on the use of a pointing device.

Designing such interaction methods means to plan the use of the fingers

which read the contents of the display and at the same time point to an

object of the application. The lack of overview which a blind user has can

be detrimental on his work. To obtain a display's the user reads over a

large part of the display. In an ideal dialog, there would be no extra

finger movement in the process of pointing.

Recent research into eye-movement based interaction methods [Jacob90] has

also shown that it is difficult to seperate - from the computers point of

view - reading and pointing. To characterize the movements of a finger on a

tactile display, we have described a syntax of reading in [Weber89]. By

defining in the following a model for the semantics that is behind such a

syntactical description we are able to seperate pointing and reading.

3. A Model for Direct, Indirect and Active Interaction

Interaction is in this context the usually non-textual part of man-machine

communication. A technical model of communication establishes channels that

deliver messages between the sender and the receiver of information. The

finger movements are messages that are sent by the user to the computer.

Since on the touch-sensitive pin-matrix device both forefingers are

monitored, two independent channels are used. The messages from the user

result in an update of the contents of the pin-matrix device. Each hand has

its own reading region and similarily as for input these are two

independent output channels. Output messages can be generated by :

- The low-level input/output system of the user interface (device drivers),

- The device independent part of the user interface, or

- The application program through the "Application Programm Interface

(API)".

By additionally taking into account the time until there is a feedback

generated, we can describe the following types of interaction.

Indirect interaction relies not on immediate feedback in response to the

finger's movement. Only the user's sensations that, for example, accompany

the flexion of the joints, are recognized and allow the user to guide the

movement. The result of the interaction is a message that is indirectly

sent to the user interface (after the movement has been evaluated).

Direct interaction necessitates immediate messages to the user interface

and in particular to the objects that are displayed. The immediate feedback

via refresh of the reading-region to the user results in additional

sensations compared to indirect interaction. The objects of the user

interface must therefore become sensitive to the position of the reading-

region.

Within both these modes of interaction, the application program is only

passive. The application program does not evaluate the movements that

belong to pointing, since this is done inside the user interface on the

basis of the porperties of the objects that are handled.

Active Interaction encompasses the application program to provide feedback.

The refresh of the reading-region is a result of messages by the

application program to the user interface that correspond to the user's

movements of the fingers. The redisplay of the finger's reading-region

therefore opens up the possibility to guide the finger's movements on the

display.

Our initial question was how to seperate reading from pointing. Of course

this can be done by involving extra steps during interaction with the

computer (e.g. press a button, wait a second).

Both, reading and pointing are with respect to the previously introduced

types of interaction different methods of interaction. Each of the types of

interaction allows different interaction methods. Reading without poiniting

is indirect interaction. The user explores the display with his hand.

Direct interaction involves both interaction methods at the same time.

Since direct interaction allows to undo operations quickly, there is no

restriction on exploring the display while dragging an icon. On the other

hand active interaction does not allow for exploration of the display while

both reading and pointing are done at the same time.

6. An application: KoFi

KoFi ("Konstruieren mit den Fingern" = "Constructing by fingers") is a

drawing program which has been implemented on a SUN 3/50 in C with the

touch-sensitive pin-matrix device as the input and output device.

Every one of the above mentioned types of interaction can be used to

construct the objects of a drawing. Construction of drawing objects usually

depends heavily on the size, position and shape of other objects of the

drawing. The user therefore must be able to select the type of interaction

that most best matches his needs of an overview of the drawing. Direct

interaction is used in KoFi for example to span a straight line with both

forefingers. Indirect interaction is typically in use when issuing a

gesture [Wolf87, Kankaanpaa88]. A gesture is a kind of movement that does

not rely on the feedback of the computer. The user can draw for example a

complete rectangle with his fingers. The size of the rectangle is evaluated

by the user interface and a corresponding message is sent to the

application program. Variations of the gesture (for example drawing it in a

clockwise or anti-clockwise manner) can be evaluated to express parameters

of the gesture [Weber87]. The specification of the gestures is done in the

language FINGER [Weber90].

Constructing a circle in KoFi by issuing a gesture with one hand enables

the user to read the contents of the display during the contruction because

leaving the ideal form of the circle is not considered a detail of

construction (see Fig. 3, crosses mark the trail of the finger).

Another form of direct interaction is in use during the construction of

freehand drawings. Only the touch-sensitive pin-matrix device currently

enables blind users to draw with their fingers while the drawing can

simultaneously be felt. In KoFi, freehand drawings cannot be redesigned

after their construction has been finished. With any other of the

constructed objects this is possible.

If a certain object is chosen by the user to be constructed, only the

starting point has to be marked and by means of active interaction, the

user can be assisted in the construction of the object.

For example, the construction of a straight line can be carried out this

way. From the starting point, a straight line is drawn by the computer to

the user's finger. If the user leaves the part of the line constructed so

far, the response to his finger's movement is a completion of the line. The

finger is then guided so as to stay along the straight line. If the users

fingers move too far from the straight line, that the reading region no

longer includes the original line, the straight line is redrawn by KoFi.

KoFi is an application with an user interface, that is designed for blind

users. It shows that there is an advantage of having a pointing device and

a graphical display in one field: that of constructing a drawing.

7. Dealing with existing user interfaces

In dealing with existing user interfaces that work with graphical displays

and pointing devices we have to ask :

- Whether there is a "translation" possible and

- Whether this "translation" allows the blind user to have the same

benefits that a sighted user has in dealing with graphical user interfaces.

Advantages should be :

- An increased speed of interaction in general

- The reduction of the complexity of objects of a user-interface by

providing "user friendly" interaction methods (evaluate usability)

- To allow the consistent use of metaphors from both "worlds", the visual

and the non-visual in collaborative envirionments.<R>

The following are examples of how to translate existing interaction

methods.

Typing on a keyboard is a kind indirect interaction. The response by the

user interface is an update of the pins, but since the fingers are on the

keyboard they cannot read at the same time. Especially MS-Windows allows to

a great extent alternatively specify a option from a menu by either typing

letter combinations or by using the mouse.

If there exists no keyboard alternative to the process of selecting a

option the common metaphor is that of pointing to it, getting an immediate

response (e.g. the colours of this option are changed) and - if desired -

selecting this option with a button press. This type of interaction is to

be called direct.

For the blind user a field in a menu is selected by reading and pointing

with the same finger. The user reads every possible alternative of the

menu. Having read the unwanted alternatives, the reading finger is already

at the desired position if the alternative currently beeing read is to be

selected. A selection is made for example by tipping onto the option.

Todays hardware for "cursor routing" is already capable of accepting such a

method of selection.

Direct manipulation of icons is another metaphor that can be performed by

the blind user with both of his hands. A common example is that of deleting

a file by moving an icon that represents the file over an icon representing

a trash can. To accomplish this one forefinger reads the contents of the

display to find the icon that is to be moved. The second forefinger

explores the display to determine the location of the trash can. The user

then drags with his first hand the icon to the position of the second hand.

During the movement, the distance between the two forefingers is

continually evaluated by the user to find the destination. When the icon

finally arrives at the trash can it will be deleted from the display.

One could try now with this in mind to make simply new device drivers for

the touch-senstive pin-matrix device in existing user-interfaces as it is

done if a track ball replaces the mouse or a certain brand of a monitor has

to be made accessable.

A good candidate for such an approach is X-Windows. This UNIX-based

software product seperates various aspects of the user interface into what

is done at the "client side" (usually the host computer) and what is done

at the "server side" (in other words the terminal). For us it is importnat

that such a server can be a computer in itself, to which the pin-matrix

device is connected.

Presentation Manager is another candidate since the OS/2 operating system

allows for multi-tasking. Thereby a special task could translate messages

to the display resp. translate the messages from the pointing device.

But there are restrictions to this approach:

- The size of the pin-matrix device with more than 7000 pins is small

compared with more than 300000 pixels of a VGA display,

- The fine details of most objects shown are not of interest to the blind

user,

- Blind users need a braille notation, or - if appropiate - speech output

instead of a tactile reproduction of ink print, and

- The blind user needs very often a different design of the interaction

methods in order to allow the exploration of the display with at least one

of the hands.

To overcome these restrictions there has to be a more abstract level of

description than having when only (coloured) pixels and coordinates of the

pointing device are involved. Such a description has to describe the

combined use of each of these messages, that is the complete interaction

method. Such a description can serve as the basis for new special user

interfaces for the blind and as a basis for an adaptation of existing user

interfaces. Future research has to show whether this can be general purpose

for several user interfaces and whether such a description can be matched

to existing interfaces used by application programs (see also

[Stephanidis90]).

8. Concluding Remarks

Interaction based on finger-movements can be used to overcome the lack of

overview of blind users. The design of dialogue systems based on tactile

representations must include different types of interaction. With the use

of both forefingers, current dialogue techniques for sighted can be adapted

for the blind. This gives the basis for the use of graphical user

interfaces by the blind.

9. Acknowledgements

I wish to thank Prof. Dr. R. Gunzenhuser and Dr. W. Schweikhardt who made

it possible to undertake this reseach project. I also want to thank Prof.

Dr. H. Jrgensen from University of Western Ontario, Canada for his

assisstance in providing me with the necessary computing resorces to write

this paper during our work on the North American pin-matrix device in

London, Ontario.

10. Literature

[Anbary82]

Anbary, Y.; Mardiks, E.: A computer terminal with a single-cell braille

display, in Raviv, J.(ed.): Uses of computers in aiding the disabled,

North-Holland, Amsterdam, 1982, pp.367-376

[Bliss70]

Bliss, J.C.; Katcher, M.H.; Rogers, C.H.; Shepard, R.P.: Optical-to-tactile

image conversion for the blind, IEEE Transactions on Man-Machine-Systems,

1970, 11, pp. 58-64

[Fischer85]

Fischer, G.; Lemke, A.; Schwab, T.: Knowledge-based help systems, in Human

Factors in Computing Systems, ACM, 1985, pp.161-167

[Gaver89]

Gaver, W.W.: The SonicFinder: An interface that uses auditory icons, Human

Computer Interaction, 4, 1989, pp.67-94

[Gibson62]

Gibson, J.J.: Observations on active touch, Psychological Review, 1962, 69,

6, pp.477-491

[Gordon78]

Gordon, G.: Active touch, Pergamon Press, Oxford, 1978

[Hill88]

Hill, D.R.; Gries, Chr.: Substitution for a restricted visual channel in

multimodal computer-human dialogue. IEEE Transactions on Systems, Man, and

Cybernetics, 18(2), pp.285-403

[Kankaanpa88]

Kankaanpaa, F.: FIDS - A flat-panel interactive display system, IEEE

Computer Graphics & Applications, 1988, pp.71-82

[Karshmer85]

Karshmer, A.; Myler, H.; Davis, R.: An inexpensive and portable talking-

tactile terminal for the visually handicapped, Computing Research

Laboratory, New Mexico State University, 1985

[NASA90]

NASA: "Braille-Maus" fr Blinde, machine design, 1, 1990, cited according

to Schnellpresse, 35, 1990

[Papenmeier]

F.H.Papenmeier GmbH &Co, KG, Talweg 2, D-5840 Schwerte

[Schweikhardt84]

Schweikhardt, W.: Representing videotex-pages to the blind, in Proc. of

Third Annual Workshop on Computers and the Handicapped, IEEE, 1984, pp.23-

29

[Stephanidis90]

Stephanidis, C.; Deconinck, F.; Krause, M.; Maderlechner, G.; Ricketts, I.;

Strothotte, Th.: Image understanding and machine vision, in Fellbaum, K.

(ed.): Access to visual computer information by blind persons, Technical

University of Berlin, Berlin, 1990

[Weber87]

Weber, G.: Gestures as a means for the blind to interact with a computer,

in Bullinger, H.-J.; Shackel, B.(eds.) : Human-Computer-Interaction,

INTERACT '87, North Holland, Amsterdam, 1987, pp.593-595

[Weber89]

Weber, G.: Interaktive Dialogtechniken f\"ur blinde Rechnerbenutzer

(Interactive dialogue techniques for blind computer users, in german),

Doctoral Thesis, University of Stuttgart, 1989

[Weber90]

Weber, G.: FINGER - A language for gesture recognition, Proceedings of

INTERACT '90, North Holland, in print

[Wihofsky90]

Wihofsky, J.: Ein mausgesteuertes Vorleseprogramm fr Blinde,

Studienarbeit, Universitt Stuttgart, Institut fr Informatik, 1990

[Wolf87]

Wolf, C.G.; Morrel-Samuels, P.: The use of hand-drawn gestures for text

editing, in Int. J. Man-Machine Studies, 1987, 27, pp.91-102