Optical Character Recognition: an overview.

James R. Fruchterman

(Transcription of the taped lecture)

I'm hoping to give you an overview of what's being transpiring in reading

machines and optical character recognition and with deference to prior

comments, I will try to keep it on a non-commercial basis and Arkenstone is

a non-profit maker of these sorts of devices.

I think the major advancements in reading technology in the last couple of

years have been in terms of reducing the cost and improving the

performances of reading systems.

I will first talk about what forms of reading systems are taken, and what

the components are, and some of the technologies that go into a reading

system, as well as following that with applications, discussing how reading

machines get used by different people in different ways and by different

institutions. And then by summing up what we may expect for the next couple

of years in reading technology, because I think it is a field that's going

rapidly involving. One of the major reasons why reading technology is

moving forth very quickly, is because it has very large commercial

applications. So, in this case, we can ride huge volumes of hundreds of

thousands of units to drive cost down by specializing these products. Now,

I talk about what kind of specializations are involved on these basic

commercial products, which have being getting very inexpensive recently.

The first part of a reading system is taking a picture of the page. This

can take several different forms and is almost always an image scanner of

some kind and probably the most general one is a flat-bed scanner. We have

a glass platen and you put a sheet of paper or a book down on the glass, it

is then scanned and you have your electronic picture. Now, what happens

after that point is what really differentiates the systems in terms of how

accurate they are. But it is important to get a good quality initial image.

The second major type of scanners are sheet-fed scanners. This is where you

can always use a cut sheet -not a book, that's bound- and pass the sheet

through a mechanism. These are much like the scanners used in fax-machines.

Because many people want to read a broad variety of documents, sheet-fed

scanners are not as widespread in reading systems. But some of the flat-bed

systems come with an add-on to add this capability.

The last type, which is of greater interest in this field than in other

fields, are hand-held scanners. There is an interest in portable reading

systems and there is an interest in a sort more direct interaction with the

page. The major technical problems in the past have been associated with

tracking a line of text. Tracking aids have been developed but overall, I

should say that they haven't satisfied the needs as much as people would

like. I think that one interesting development, that just occured recently,

is a full-page-width scanner from Mitsubishi, which is probably going the

lead to a hand-scanner with much less problems of tracking the page. The

user will simply go down the page once by pulling a bar from the top to the

bottom.

So I think those are the ways that the systems acquire the image of the

page.

The next segment is the optical character recognition portion of a reading

system, where that pictures are turned into an understanding of the

letters. So that it can easily given to a user, either in hardcopy braille

or soft copy braille or voice synthesis.

The OCR technologies have advanced very quickly because OCR is a technology

that is not complete. It is not perfect by any means and where it is

limited, is that for every existing OCR product, it has errors, it makes

mistakes. And the answer to fixing those mistakes are to put it on a bigger

computer and write more programs.

Since computing power is gotten so cheap or it is continuing to get cheap

very rapidly, we are going to see rapid advances in OCR that track the

advances in processing power. Because OCR relies so much on processing

power, we see cost numbers falling from systems that cost 40.000 USD four

years ago, now being available for 500 USD. It is a software packet that

runs on a PC, as fast and more accurate. So that's an incredible rate of

price/performance improvements. I think we will contribute to make some

more important aid for the visually impaired.

The three types of OCR:

A. Fixed font OCR, where the machine knows certain types of fonts (ie

Courier) and recognizes that very well. And that technology was very

important in the seventies but has fallen by the weight sized as more

advanced technologies have occured.

B. OCR with trainable fonts.

The trainable OCR devices where the system is thought by a user to read a

certain type font. This was for instance the basic technology behind the

Kurzweil system, which was the first reading system and the one that has

been most widely used over the past ten years. In the case of blind users

they had the ability to sort of make a "best guess" since the blind user

couldn't examine an image of a character and train them.

C. Omni-font OCR.

They come pre-trained to recognize a wide variety of fonts. These systems

have been increasingly dominating the market and for instance Kurzweil has

an omni-font system and there are probably another couple leading ones in

the United States that I am aware of.

And then certain European systems that combine omni-font and trainable

character recognition together, so that you can take a system and improve

it. And these are the systems that occur most frequently in reading

machines.

There are probably five vendors that provide 95% of the OCR technology that

goes in reading machines, under many different names.

The output part of the reading system is something most of the audience it

is very familiar with. For an individual in the U.S it is typical voice-

synthesis because it is so inexpensive. In Europe, much more frequently

brailledisplays (soft-copy braille) and then also institutions use reading

systems to generate braille, less expensively than manual input.

That completes the transfer of the information to the user. The one other

major media that people are using is sending scanned documents by floppy.

As Computerized Books For the Blind (CBFB) does in U.S. by publishing

books, not printed braille, but on floppy for PC users.

Lastly, the system ends up either in a PC, which gets the benefit of also

the great costs advantages of PC's. Or is a stand-alone system. And an

example of a stand-alone system is the Kurzweil Personal Reader (KPR),

which is not a computer and has the advantage that to a user who doesn't

want to touch a computer, this is a reading appliance.

There is also a product, called the "Personal Companion" in the United

States that I have seen, that is a voice in, voice out system, no display,

no keyboard. It has user's specific voice recognition, it teaches itself to

understand you. Then you can have a number of functions and also have

papers be read and spoken without touching a keyboard which, I think, some

day may represent the way we all like to do with reading machines. But,

right now, it's over 10.000 USD, so it stays out of the range of most

people.

I think that when we talk about applications of reading systems, well, I

look first at the individual applications of reading systems.

In all cases, the person is faced with evaluating the options available to

them. For instance, for personally use, if all the personal information you

wanted, was available in braille form in your braille reader, as mister

Gladstone has just described the service. I think you probably rather have

it in that form than use a reading machine. Because a reading machine will

make mistakes and take a certain amount of time before you able to get at

the information. Likewise, we talk about reading a novel, it is much more

pleasant to listen to a nice , well-margelated reader, talking a book into

a tape, than listen to a mechanical voice intoning it (monotone).

In any event, I think the personal applications are mainly in the home, in

education, and employment. In the home, I think the objectives are just the

ones that mister Gladstone annunciated: privacy, an access in personal

documents, credit card statements, bank statements, phone bills and alike.

And I think there is a great amount of feedback from the users of reading

machines on me . It may seem trivial that they can get now this information

themselves, but it is very important, it is something that sighted people

take for granted and now with this tool, the reading machines user can get

at the same information. And also reading other materials that is just of

the every day nature of things that we read: recipes, magazines, and alike.

In the educational arena, I think the major applications of reading

machines have been at the university and graduate level. The fact that many

of those courses do not have accessible materials -unlike secondary and

primary schools- means that having a reading machine may make the

difference between a person pursue of graduate program or not. I think that

a very good area of reading machine development is to handle the daily

hand-outs that were prepared yesterday and need to be ready for tomorrow.

Lastly employment: I think that reading machines complement the increasing

availability of computer-based employment opportunities. With a computer, a

blind person can do anything a sighted person can in a typical application

and a reading machine can fill in the rest of the environment. It helps the

person access, memo's that are not available on the computer and alike.

I think braille production is a major application of reading machines, and

it parallels to commercial application of replacing keyboards and getting

information into the computer.

So, I think that's a rapid overview of what is happening in reading

machines. I think that when we look at the most important issues of reading

systems, certainly we at Arkenstone are dedicated to bring the prices down

of reading systems. And that -we think- is a major thing to the use of this

technology. For instance, as a group in the U.S., reading systems account

from over 10.000 USD, 18 months ago, to being in the 2.500 USD to 6.000 USD

range for PC-systems. I think that those cost trends will continue and I

think there is a goal -certainly in our minds- of reducing that price to

less than a 1.000 USD for reading-technology within the next 2 or 3 years,

that will read better and better than current systems.

So I think that cost is the number one thing in making reading technology

more available. The second thing is making the reading technology more

accessible to the user, as a person, the human-factor issues.

Today, certainly at Arkenstone, our emphasis is on providing PC-based

reading systems. And the majority of the sighted population does not want

to touch a PC. I think that blind users are just the same as everyone else.

I think there is a trend to continue improve the computer based systems but

also to make the reading systems more accessible and less expensive to a

person, so that a reading system intrinsically is like a cane. Something

you can just learn to use, you don't think twice about it and it just does

what it needs to do and all those wonderful techy details how it does it,

are your concern as a user. I think that is an important goal for reading

systems. So I think that by delivering those two aspects we will see

reading machines becoming much more prevalent.

Thank you.