IBM workstation for the blind people.

Claude Ronsse.

1. Personal Experience

My first contact with IBM was in 1975. IBM had created a non-profit

association, called AIDP (Association for Integration in Data Processing),

which mission was to organize programmation courses of 1,5 year for

disabled persons. We had theoretical education, practice exercises and a

training period in other companies.

The courses were open to all disabled people, independently their handicap.

They were given in the IBM International Education Center, in La Hulpe

(near to Brussels). The content of the course was the learning of computer

languages, such as Cobol, Assembler, PLI, as well as TP communication. At

that time the Optacon was the reading aid for blind people. It was used to

read listings and punched cards. One of the problems we met was the reading

of the documentation.

The Optacon that was then used, was a camera that was positioned on a

document and that transmitted the image to a tactile plate. This plate was

equipped with vibrating pins which permitted the blind person to feel what

was read by the camera. In that way, it was possible to read, character by

character, a message or program statement.

After this course, I have been hired by IBM as a programmer. I started in

the internal computer application development department.

In that time, I had a braille printer to print out programs, reference

listings, phone directory, etc.

As everybody in that time, I switched from the punched cards to a screen.

In a first time, I used the Optacon. Later on, IBM announced an audio

terminal connected to the main frame.

I then used both Optacon and audio screen, depending on their respective

performances.

When leaving that department, I was Systems Analyst, in charge of the

development and maintenance of the Personnel Department applications.

Since two years, I am now working at the IBM European Support Center for

Persons with Disabilities where I am in charge of the products destinated

to bisually impaired people.

I not longer need several devices to do my job. One is sufficient, and it's

called :

2. The IBM Screen Reader

Screen Reader is an audio system composed of : a software loaded into the

memory of the IBM PS/2, an 18-key keypad and a synthesiser. IBM has choosen

for an audio-system after having found out that only 10 percent of the

blind american population knows braille. Also a large majority of blind

customers and IBM employees, preferred an audio system as communication

tool.

Audio is also a more natural way to communicate. There exists on the market

a big choise of synthesisers of different types and with a very large price

scale.

IBM has thus decided to develop the Screen Reader as a standard product

working on a standard machine and running on standard applications. Screen

Reader runs on the complete IBM PS/2 family, as well as on PC XT and AT.

The Screen Reader standard program runs under DOS.

There is no direct link between Screen Reader and the running applications.

This means that all the applications used with Screen Reader are standard.

The communication bridge between Screen Reader and the application is the

image of the screen in DOS.

The concept of the Screen Reader is to give the blind user an audio

feedback of what happens on the screen. In using the keypad, the user can

send reading requests to the software, asking it to read a character, a

word, a line, the whole screen, sentence by sentence, a message, an opened

window on the screen, etc. There exists the possibility to interrupt the

reading each time it's necessary, for instance, to spell a word, to repeat

the line, the word, the paragraph and to restart the reading on the

position where it was interrupted.

During typing, Screen Reader can automatically repeat the word, character,

line that has just been typed by the user.

On top of these basic functions, the main interest of the software is the

way it can be adapted to an application. Screen Reader gives an

automatically selected feedback of the changes on the screen.

Automatically means that, while typing on the standard keyboard, the blind

user receives the screen information in parallel, and althus works in the

same way as a sighted person does.

Selected feedback means that Screen Reader gives just the information

needed to control the application. When a sighted person works with an

application, he does not read the entire menu that appears on the screen;

he only reads the check points. Screen Reader can be programmed in the same

way. The blind user can thus follow, in parallel, the result of his

commands while typing.

Besides the standard facilities, Screen Reader can be adapted so that it

follows and reads all messages prompted on a screen. This offers the

possibility to work with applications, that open several windows on the

screen, in reducing the size of its audio window, and so permit to read a

window, and to jump from one window to another one.

Screen Reader also works with selection menus which highlight the active

function with colour change, instead of working with the position of the

cursor.

3. There exist three adaptation levels :

The first one is "preprogrammed profiles". A profile is an additional

automatism that is added to the standard ones and specifically adapted to

an application.

Several profiles are delivered with the package. They are adapted to

standard softwares like text editors, such as Writing Assistant,

DisplayWrite, Word Perfect, DBase, etc., but also to spreadsheet handlers

like Lotus or emulation programs. It is very easy to switch from one to

another in usinq the keypad or in typing a DOS command.

A second tailoring level is done in using the Screen Reader keypad. It is

possible, in an easy way and in using the keypad, to ask Screen Reader

- to control the appearance of messages on the screen,

- to control the opening of windows,

- to follow automatically a selection of functions made on a line or in a

menu by colour highlighting.

The user can also reduce the size of the audio window at any moment when,

for instance, he is working with columns, etc.

The third level of adaptation is the use of a powerful computer language

called Profile Access Language. It authorises the user to define his own

automatisms and to redefine all or part of the keypad functions.

The sources of the existing profiles are provided with the package and can

be updated or serve as sample.

4. The keypad.

The keypad is connected to the standard mouse entry of the PS/2. It can be

easily moved from a PS/2 to another one. IBM choose to work with a

separated keypad for several reasons.

The first one is to avoid any confusion. Through the keypad, the user

communicates with the Screen Reader Audio System, throught the keyboard he

communicates with the running application. It also makes the Screen Reader

independent of other applications. If the PS/2 keyboard had to be used as

application keyboard and as Screen Reader one, it should have needed a key

to switch from the application mode to Screen Reader mode. This would have

resulted into the impossibility to use the audio system in the same time as

working with the running application.

It is also easier for a blind person to r* er the keys' functions of an 18-

keys keypad rather than those of a keyboard where each key has several

functions attributed.

5. The synthesiser.

The synthesiser is not an IBM product. It is connected to the PS/2 using

the RS232 connection slot which is an industrial standard, and thus more

than an IBM one.

We choose to work with an independent synthesiser to avoid that the

applications running on the PS/2 be impacted and that the response time be

affected.

Card synthesisers need a specific program adaptation due to the fact that

they do not use a slot to communicate with Screen Reader. IBM is ready to

provide the source of the Screen Reader communication program so that the

synthesiser card manufacturers can adapt their proqrams.

The multiple language synthesisers are a must. More and more, documentation

updatings are provided in English. Inter-company communications are growing

and are done in English. This results in a need for the blind employee to

dispose on his national language and at least the English one.

Using this type of synthesisers makes it possible to switch from one

language to another just in pressing keys on the Screen Reader keypad.

6. Accessible applications

It is possible to run stand-alone applications on the PS/2, but this one

can also be connected to a bigger system and thus serve as a terminal. This

means that Screen Reader can not only work with agendas, phone directories,

spreadsheets, like Lotus, with means accounting, budget, planning, ... but

also with standard applications running on a network, for instance

electronic mail or other ones, specific to the enterprise.

7. Connection to PS/2

Screen Reader works on a standard IBM PS/2 computer which means that a lot

of other products can be connected to that PS/2, such as :

- a normal printer so that the blind user can print texts that have to be

read by other people.

- a braille printer to have the possibility of receiving, at any moment, a

Braille hard copy of notes, documentation, ...

- a phone connection, network connection coax or token ring, for electronic

mail and shared applications.

- a scanner, for instance the IBM 3119 and an OCR software.

The scanner will recognize printed paper text, letters, pages of a book,

and will prompt it on the screen so that, in using the Screen Reader, the

blind person can read them and print them out in Braille. The blind person

can also use Screen Reader to control the scanning program (parameters and

messages). Therefore, we need a software which accepts automatically a

large scale of characters and automatisms such as swapping of pictures

appearing on the paper.

8. My job.

In IBM, I am responsible for the support of the Screen Reader throughout

Europe. I am also in charge of the products for visually impaired people.

Therefore, I am constantly in contact with our European counterparts as

well as with our development laboratories in the USA.

I use the Screen Reader on an IBM PS/2 which is connected to the internal

IBM network. This offers me the possibility to send notes to IBM people,

wherever they are working in the world. I also have a scanner. If I want to

read documentation on a product, I scan the text and read it with the

Screen Reader.

If I need a hard copy in Braille, for example to have notes at my disposal

during a meeting, I can print them on my Braille printer which is also

connected to my PS/2.

Because Screen Reader is standard and works on a standard IBM PS/2, I have

to take with me only the small keypad, a diskette and a synthesiser, when I

make demonstrations during international events.